# Supplementary Material

Valid prediction intervals for transferred NIR calibrations: reusing the transfer standards with a leave-one-standard-out jackknife+

All tables and figures in this document were generated automatically from the machine-readable result files of the study (results/).

# Table S1. Main results for every scenario

Table S1. Coverage of nominal 90% intervals and median interval score for every primary scenario (α = 0.10). CI: percentile bootstrap over repetitions.

| Data | Response | Target | m | Method | Coverage [95% CI] | Median IS | Under-covering |
| --- | --- | --- | --- | --- | --- | --- | --- |
| corn | moisture | mp5 | 10 | ASTM-PI | 0.035 [0.032, 0.039] | 4.71 | True |
| corn | moisture | mp5 | 10 | Gauss-LOO | 0.897 [0.888, 0.907] | 1.36 | False |
| corn | moisture | mp5 | 10 | SCP-S | 0.063 [0.059, 0.068] | 4.54 | True |
| corn | moisture | mp5 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | moisture | mp5 | 10 | TJ+ | 0.919 [0.909, 0.927] | 1.42 | False |
| corn | moisture | mp5 | 10 | TJ+N | 0.922 [0.912, 0.930] | 1.48 | False |
| corn | moisture | mp5 | 10 | Transport | 0.386 [0.361, 0.410] | 3.06 | True |
| corn | moisture | mp5 | 10 | WCP-S | 0.306 [0.292, 0.322] | ∞ | True |
| corn | moisture | mp5 | 15 | ASTM-PI | 0.037 [0.033, 0.042] | 4.59 | True |
| corn | moisture | mp5 | 15 | Gauss-LOO | 0.893 [0.883, 0.901] | 1.29 | False |
| corn | moisture | mp5 | 15 | SCP-S | 0.064 [0.059, 0.070] | 4.39 | True |
| corn | moisture | mp5 | 15 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | moisture | mp5 | 15 | TJ+ | 0.942 [0.934, 0.949] | 1.4 | False |
| corn | moisture | mp5 | 15 | TJ+N | 0.934 [0.926, 0.941] | 1.41 | False |
| corn | moisture | mp5 | 15 | Transport | 0.332 [0.309, 0.354] | 3.12 | True |
| corn | moisture | mp5 | 15 | WCP-S | 0.220 [0.208, 0.233] | ∞ | True |
| corn | moisture | mp5 | 20 | ASTM-PI | 0.035 [0.030, 0.040] | 4.32 | True |
| corn | moisture | mp5 | 20 | Gauss-LOO | 0.907 [0.896, 0.917] | 1.19 | False |
| corn | moisture | mp5 | 20 | SCP-S | 0.068 [0.061, 0.076] | 4.14 | True |
| corn | moisture | mp5 | 20 | SCP-T-split-N | 0.914 [0.903, 0.924] | 1.41 | False |
| corn | moisture | mp5 | 20 | TJ+ | 0.915 [0.905, 0.925] | 1.22 | False |
| corn | moisture | mp5 | 20 | TJ+N | 0.911 [0.902, 0.920] | 1.26 | False |
| corn | moisture | mp5 | 20 | Transport | 0.307 [0.286, 0.327] | 2.99 | True |
| corn | moisture | mp5 | 20 | WCP-S | 0.204 [0.190, 0.217] | ∞ | True |
| corn | moisture | mp6 | 10 | ASTM-PI | 0.026 [0.023, 0.029] | 5.32 | True |
| corn | moisture | mp6 | 10 | Gauss-LOO | 0.903 [0.893, 0.913] | 1.52 | False |
| corn | moisture | mp6 | 10 | SCP-S | 0.051 [0.047, 0.055] | 5.12 | True |
| corn | moisture | mp6 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | moisture | mp6 | 10 | TJ+ | 0.919 [0.910, 0.929] | 1.6 | False |
| corn | moisture | mp6 | 10 | TJ+N | 0.918 [0.909, 0.927] | 1.61 | False |
| corn | moisture | mp6 | 10 | Transport | 0.366 [0.342, 0.392] | 3.48 | True |
| corn | moisture | mp6 | 10 | WCP-S | 0.303 [0.288, 0.318] | ∞ | True |
| corn | moisture | mp6 | 15 | ASTM-PI | 0.032 [0.028, 0.036] | 5.12 | True |
| corn | moisture | mp6 | 15 | Gauss-LOO | 0.898 [0.888, 0.907] | 1.38 | False |
| corn | moisture | mp6 | 15 | SCP-S | 0.056 [0.051, 0.061] | 4.94 | True |
| corn | moisture | mp6 | 15 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | moisture | mp6 | 15 | TJ+ | 0.941 [0.933, 0.949] | 1.51 | False |
| corn | moisture | mp6 | 15 | TJ+N | 0.935 [0.927, 0.942] | 1.53 | False |
| corn | moisture | mp6 | 15 | Transport | 0.313 [0.291, 0.335] | 3.57 | True |
| corn | moisture | mp6 | 15 | WCP-S | 0.223 [0.210, 0.237] | ∞ | True |
| corn | moisture | mp6 | 20 | ASTM-PI | 0.033 [0.028, 0.038] | 4.83 | True |
| corn | moisture | mp6 | 20 | Gauss-LOO | 0.906 [0.896, 0.916] | 1.28 | False |
| corn | moisture | mp6 | 20 | SCP-S | 0.058 [0.051, 0.065] | 4.68 | True |
| corn | moisture | mp6 | 20 | SCP-T-split-N | 0.912 [0.902, 0.922] | 1.53 | False |
| corn | moisture | mp6 | 20 | TJ+ | 0.910 [0.900, 0.920] | 1.28 | False |
| corn | moisture | mp6 | 20 | TJ+N | 0.909 [0.899, 0.918] | 1.38 | False |
| corn | moisture | mp6 | 20 | Transport | 0.288 [0.267, 0.310] | 3.37 | True |
| corn | moisture | mp6 | 20 | WCP-S | 0.195 [0.181, 0.208] | ∞ | True |
| corn | oil | mp5 | 10 | ASTM-PI | 0.184 [0.175, 0.195] | 1.35 | True |
| corn | oil | mp5 | 10 | Gauss-LOO | 0.911 [0.903, 0.920] | 0.597 | False |
| corn | oil | mp5 | 10 | SCP-S | 0.453 [0.438, 0.467] | 0.874 | True |
| corn | oil | mp5 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | oil | mp5 | 10 | TJ+ | 0.951 [0.945, 0.958] | 0.667 | False |
| corn | oil | mp5 | 10 | TJ+N | 0.948 [0.941, 0.954] | 0.637 | False |
| corn | oil | mp5 | 10 | Transport | 0.884 [0.872, 0.895] | 0.651 | True |
| corn | oil | mp5 | 10 | WCP-S | 0.590 [0.573, 0.605] | ∞ | True |
| corn | oil | mp5 | 15 | ASTM-PI | 0.211 [0.200, 0.222] | 1.22 | True |
| corn | oil | mp5 | 15 | Gauss-LOO | 0.906 [0.898, 0.914] | 0.482 | False |
| corn | oil | mp5 | 15 | SCP-S | 0.500 [0.486, 0.514] | 0.777 | True |
| corn | oil | mp5 | 15 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | oil | mp5 | 15 | TJ+ | 0.957 [0.950, 0.963] | 0.556 | False |
| corn | oil | mp5 | 15 | TJ+N | 0.951 [0.944, 0.958] | 0.523 | False |
| corn | oil | mp5 | 15 | Transport | 0.876 [0.864, 0.889] | 0.556 | True |
| corn | oil | mp5 | 15 | WCP-S | 0.581 [0.565, 0.597] | 2.92 | True |
| corn | oil | mp5 | 20 | ASTM-PI | 0.208 [0.196, 0.221] | 1.22 | True |
| corn | oil | mp5 | 20 | Gauss-LOO | 0.903 [0.894, 0.912] | 0.476 | False |
| corn | oil | mp5 | 20 | SCP-S | 0.519 [0.503, 0.535] | 0.745 | True |
| corn | oil | mp5 | 20 | SCP-T-split-N | 0.915 [0.905, 0.925] | 0.53 | False |
| corn | oil | mp5 | 20 | TJ+ | 0.917 [0.909, 0.926] | 0.47 | False |
| corn | oil | mp5 | 20 | TJ+N | 0.921 [0.913, 0.929] | 0.447 | False |
| corn | oil | mp5 | 20 | Transport | 0.881 [0.869, 0.894] | 0.54 | True |
| corn | oil | mp5 | 20 | WCP-S | 0.593 [0.577, 0.609] | 0.911 | True |
| corn | oil | mp6 | 10 | ASTM-PI | 0.201 [0.190, 0.211] | 1.27 | True |
| corn | oil | mp6 | 10 | Gauss-LOO | 0.901 [0.890, 0.911] | 0.596 | False |
| corn | oil | mp6 | 10 | SCP-S | 0.473 [0.458, 0.488] | 0.832 | True |
| corn | oil | mp6 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | oil | mp6 | 10 | TJ+ | 0.946 [0.939, 0.953] | 0.642 | False |
| corn | oil | mp6 | 10 | TJ+N | 0.947 [0.940, 0.953] | 0.612 | False |
| corn | oil | mp6 | 10 | Transport | 0.921 [0.912, 0.930] | 0.679 | False |
| corn | oil | mp6 | 10 | WCP-S | 0.610 [0.595, 0.626] | ∞ | True |
| corn | oil | mp6 | 15 | ASTM-PI | 0.215 [0.203, 0.226] | 1.18 | True |
| corn | oil | mp6 | 15 | Gauss-LOO | 0.913 [0.904, 0.921] | 0.49 | False |
| corn | oil | mp6 | 15 | SCP-S | 0.517 [0.501, 0.533] | 0.724 | True |
| corn | oil | mp6 | 15 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | oil | mp6 | 15 | TJ+ | 0.961 [0.956, 0.967] | 0.574 | False |
| corn | oil | mp6 | 15 | TJ+N | 0.959 [0.954, 0.965] | 0.551 | False |
| corn | oil | mp6 | 15 | Transport | 0.920 [0.910, 0.930] | 0.582 | False |
| corn | oil | mp6 | 15 | WCP-S | 0.600 [0.583, 0.616] | 3.27 | True |
| corn | oil | mp6 | 20 | ASTM-PI | 0.229 [0.216, 0.243] | 1.15 | True |
| corn | oil | mp6 | 20 | Gauss-LOO | 0.906 [0.897, 0.915] | 0.461 | False |
| corn | oil | mp6 | 20 | SCP-S | 0.538 [0.522, 0.554] | 0.711 | True |
| corn | oil | mp6 | 20 | SCP-T-split-N | 0.914 [0.904, 0.924] | 0.51 | False |
| corn | oil | mp6 | 20 | TJ+ | 0.924 [0.916, 0.932] | 0.468 | False |
| corn | oil | mp6 | 20 | TJ+N | 0.927 [0.919, 0.934] | 0.444 | False |
| corn | oil | mp6 | 20 | Transport | 0.924 [0.914, 0.934] | 0.569 | False |
| corn | oil | mp6 | 20 | WCP-S | 0.614 [0.597, 0.632] | 1.08 | True |
| corn | protein | mp5 | 10 | ASTM-PI | 0.381 [0.363, 0.399] | 1.77 | True |
| corn | protein | mp5 | 10 | Gauss-LOO | 0.906 [0.897, 0.915] | 0.92 | False |
| corn | protein | mp5 | 10 | SCP-S | 0.618 [0.599, 0.636] | 1.07 | True |
| corn | protein | mp5 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | protein | mp5 | 10 | TJ+ | 0.933 [0.925, 0.940] | 0.991 | False |
| corn | protein | mp5 | 10 | TJ+N | 0.926 [0.919, 0.934] | 1.02 | False |
| corn | protein | mp5 | 10 | Transport | 0.883 [0.872, 0.894] | 0.964 | True |
| corn | protein | mp5 | 10 | WCP-S | 0.709 [0.693, 0.725] | ∞ | True |
| corn | protein | mp5 | 15 | ASTM-PI | 0.413 [0.395, 0.432] | 1.52 | True |
| corn | protein | mp5 | 15 | Gauss-LOO | 0.909 [0.900, 0.917] | 0.811 | False |
| corn | protein | mp5 | 15 | SCP-S | 0.667 [0.651, 0.684] | 0.93 | True |
| corn | protein | mp5 | 15 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | protein | mp5 | 15 | TJ+ | 0.951 [0.945, 0.958] | 0.927 | False |
| corn | protein | mp5 | 15 | TJ+N | 0.944 [0.938, 0.951] | 0.943 | False |
| corn | protein | mp5 | 15 | Transport | 0.897 [0.886, 0.908] | 0.839 | False |
| corn | protein | mp5 | 15 | WCP-S | 0.727 [0.709, 0.742] | 1.71 | True |
| corn | protein | mp5 | 20 | ASTM-PI | 0.419 [0.399, 0.440] | 1.48 | True |
| corn | protein | mp5 | 20 | Gauss-LOO | 0.909 [0.900, 0.919] | 0.745 | False |
| corn | protein | mp5 | 20 | SCP-S | 0.670 [0.651, 0.689] | 0.875 | True |
| corn | protein | mp5 | 20 | SCP-T-split-N | 0.910 [0.899, 0.920] | 0.945 | False |
| corn | protein | mp5 | 20 | TJ+ | 0.915 [0.906, 0.925] | 0.774 | False |
| corn | protein | mp5 | 20 | TJ+N | 0.917 [0.908, 0.926] | 0.793 | False |
| corn | protein | mp5 | 20 | Transport | 0.898 [0.886, 0.910] | 0.811 | False |
| corn | protein | mp5 | 20 | WCP-S | 0.720 [0.702, 0.738] | 1.28 | True |
| corn | protein | mp6 | 10 | ASTM-PI | 0.393 [0.374, 0.411] | 1.66 | True |
| corn | protein | mp6 | 10 | Gauss-LOO | 0.910 [0.901, 0.919] | 0.886 | False |
| corn | protein | mp6 | 10 | SCP-S | 0.626 [0.608, 0.644] | 1.03 | True |
| corn | protein | mp6 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | protein | mp6 | 10 | TJ+ | 0.942 [0.935, 0.948] | 0.968 | False |
| corn | protein | mp6 | 10 | TJ+N | 0.933 [0.926, 0.939] | 0.999 | False |
| corn | protein | mp6 | 10 | Transport | 0.913 [0.903, 0.924] | 0.964 | False |
| corn | protein | mp6 | 10 | WCP-S | 0.730 [0.714, 0.746] | ∞ | True |
| corn | protein | mp6 | 15 | ASTM-PI | 0.427 [0.408, 0.446] | 1.46 | True |
| corn | protein | mp6 | 15 | Gauss-LOO | 0.907 [0.899, 0.915] | 0.805 | False |
| corn | protein | mp6 | 15 | SCP-S | 0.670 [0.653, 0.686] | 0.936 | True |
| corn | protein | mp6 | 15 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | protein | mp6 | 15 | TJ+ | 0.955 [0.949, 0.960] | 0.904 | False |
| corn | protein | mp6 | 15 | TJ+N | 0.950 [0.943, 0.956] | 0.934 | False |
| corn | protein | mp6 | 15 | Transport | 0.913 [0.902, 0.923] | 0.873 | False |
| corn | protein | mp6 | 15 | WCP-S | 0.733 [0.717, 0.750] | 1.77 | True |
| corn | protein | mp6 | 20 | ASTM-PI | 0.421 [0.401, 0.442] | 1.42 | True |
| corn | protein | mp6 | 20 | Gauss-LOO | 0.906 [0.897, 0.916] | 0.746 | False |
| corn | protein | mp6 | 20 | SCP-S | 0.682 [0.664, 0.700] | 0.894 | True |
| corn | protein | mp6 | 20 | SCP-T-split-N | 0.901 [0.890, 0.911] | 0.896 | False |
| corn | protein | mp6 | 20 | TJ+ | 0.920 [0.911, 0.929] | 0.775 | False |
| corn | protein | mp6 | 20 | TJ+N | 0.920 [0.911, 0.929] | 0.783 | False |
| corn | protein | mp6 | 20 | Transport | 0.908 [0.897, 0.919] | 0.849 | False |
| corn | protein | mp6 | 20 | WCP-S | 0.738 [0.720, 0.755] | 1.21 | True |
| corn | starch | mp5 | 10 | ASTM-PI | 0.273 [0.259, 0.288] | 4.88 | True |
| corn | starch | mp5 | 10 | Gauss-LOO | 0.900 [0.890, 0.910] | 2.37 | False |
| corn | starch | mp5 | 10 | SCP-S | 0.567 [0.551, 0.584] | 2.98 | True |
| corn | starch | mp5 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | starch | mp5 | 10 | TJ+ | 0.938 [0.930, 0.945] | 2.64 | False |
| corn | starch | mp5 | 10 | TJ+N | 0.932 [0.925, 0.940] | 2.45 | False |
| corn | starch | mp5 | 10 | Transport | 0.871 [0.860, 0.882] | 2.55 | True |
| corn | starch | mp5 | 10 | WCP-S | 0.708 [0.692, 0.724] | ∞ | True |
| corn | starch | mp5 | 15 | ASTM-PI | 0.282 [0.266, 0.297] | 4.61 | True |
| corn | starch | mp5 | 15 | Gauss-LOO | 0.904 [0.895, 0.913] | 2.12 | False |
| corn | starch | mp5 | 15 | SCP-S | 0.603 [0.587, 0.619] | 2.79 | True |
| corn | starch | mp5 | 15 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | starch | mp5 | 15 | TJ+ | 0.946 [0.939, 0.953] | 2.53 | False |
| corn | starch | mp5 | 15 | TJ+N | 0.948 [0.941, 0.954] | 2.37 | False |
| corn | starch | mp5 | 15 | Transport | 0.863 [0.850, 0.875] | 2.25 | True |
| corn | starch | mp5 | 15 | WCP-S | 0.705 [0.690, 0.720] | ∞ | True |
| corn | starch | mp5 | 20 | ASTM-PI | 0.290 [0.274, 0.306] | 4.44 | True |
| corn | starch | mp5 | 20 | Gauss-LOO | 0.907 [0.898, 0.917] | 1.98 | False |
| corn | starch | mp5 | 20 | SCP-S | 0.619 [0.601, 0.636] | 2.55 | True |
| corn | starch | mp5 | 20 | SCP-T-split-N | 0.908 [0.897, 0.918] | 2.28 | False |
| corn | starch | mp5 | 20 | TJ+ | 0.910 [0.901, 0.919] | 2.03 | False |
| corn | starch | mp5 | 20 | TJ+N | 0.909 [0.899, 0.919] | 1.95 | False |
| corn | starch | mp5 | 20 | Transport | 0.862 [0.850, 0.874] | 2.06 | True |
| corn | starch | mp5 | 20 | WCP-S | 0.699 [0.683, 0.716] | 5.04 | True |
| corn | starch | mp6 | 10 | ASTM-PI | 0.269 [0.255, 0.283] | 5.01 | True |
| corn | starch | mp6 | 10 | Gauss-LOO | 0.894 [0.883, 0.904] | 2.41 | False |
| corn | starch | mp6 | 10 | SCP-S | 0.559 [0.541, 0.576] | 3.19 | True |
| corn | starch | mp6 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | starch | mp6 | 10 | TJ+ | 0.928 [0.919, 0.936] | 2.6 | False |
| corn | starch | mp6 | 10 | TJ+N | 0.926 [0.918, 0.935] | 2.47 | False |
| corn | starch | mp6 | 10 | Transport | 0.893 [0.883, 0.903] | 2.48 | False |
| corn | starch | mp6 | 10 | WCP-S | 0.700 [0.685, 0.716] | ∞ | True |
| corn | starch | mp6 | 15 | ASTM-PI | 0.292 [0.276, 0.307] | 4.63 | True |
| corn | starch | mp6 | 15 | Gauss-LOO | 0.902 [0.894, 0.910] | 2.14 | False |
| corn | starch | mp6 | 15 | SCP-S | 0.606 [0.590, 0.622] | 2.79 | True |
| corn | starch | mp6 | 15 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| corn | starch | mp6 | 15 | TJ+ | 0.953 [0.947, 0.958] | 2.52 | False |
| corn | starch | mp6 | 15 | TJ+N | 0.951 [0.944, 0.957] | 2.41 | False |
| corn | starch | mp6 | 15 | Transport | 0.893 [0.883, 0.903] | 2.31 | False |
| corn | starch | mp6 | 15 | WCP-S | 0.700 [0.684, 0.715] | ∞ | True |
| corn | starch | mp6 | 20 | ASTM-PI | 0.294 [0.278, 0.310] | 4.38 | True |
| corn | starch | mp6 | 20 | Gauss-LOO | 0.904 [0.894, 0.914] | 1.95 | False |
| corn | starch | mp6 | 20 | SCP-S | 0.616 [0.600, 0.633] | 2.55 | True |
| corn | starch | mp6 | 20 | SCP-T-split-N | 0.904 [0.894, 0.914] | 2.25 | False |
| corn | starch | mp6 | 20 | TJ+ | 0.908 [0.898, 0.917] | 2.02 | False |
| corn | starch | mp6 | 20 | TJ+N | 0.908 [0.898, 0.917] | 1.93 | False |
| corn | starch | mp6 | 20 | Transport | 0.881 [0.868, 0.893] | 2.09 | True |
| corn | starch | mp6 | 20 | WCP-S | 0.696 [0.680, 0.712] | 5.21 | True |
| mango | dry_matter | S4 | 10 | ASTM-PI | 0.726 [0.720, 0.731] | 5.27 | True |
| mango | dry_matter | S4 | 10 | Gauss-LOO | 0.904 [0.892, 0.914] | 5.02 | False |
| mango | dry_matter | S4 | 10 | SCP-S | 0.707 [0.701, 0.712] | 5.43 | True |
| mango | dry_matter | S4 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| mango | dry_matter | S4 | 10 | TJ+ | 0.910 [0.899, 0.921] | 5.15 | False |
| mango | dry_matter | S4 | 10 | TJ+N | 0.910 [0.899, 0.921] | 5.06 | False |
| mango | dry_matter | S4 | 10 | WCP-S | 0.715 [0.710, 0.720] | ∞ | True |
| mango | dry_matter | S4 | 20 | ASTM-PI | 0.749 [0.746, 0.751] | 4.94 | True |
| mango | dry_matter | S4 | 20 | Gauss-LOO | 0.903 [0.895, 0.911] | 4.5 | False |
| mango | dry_matter | S4 | 20 | SCP-S | 0.730 [0.727, 0.733] | 5.09 | True |
| mango | dry_matter | S4 | 20 | SCP-T-split-N | 0.905 [0.894, 0.915] | 4.89 | False |
| mango | dry_matter | S4 | 20 | TJ+ | 0.903 [0.894, 0.910] | 4.54 | False |
| mango | dry_matter | S4 | 20 | TJ+N | 0.902 [0.893, 0.909] | 4.46 | False |
| mango | dry_matter | S4 | 20 | WCP-S | 0.738 [0.735, 0.740] | ∞ | True |
| mango | dry_matter | S4 | 30 | ASTM-PI | 0.755 [0.753, 0.757] | 4.84 | True |
| mango | dry_matter | S4 | 30 | Gauss-LOO | 0.901 [0.895, 0.907] | 4.39 | False |
| mango | dry_matter | S4 | 30 | SCP-S | 0.736 [0.733, 0.738] | 4.98 | True |
| mango | dry_matter | S4 | 30 | SCP-T-split-N | 0.933 [0.925, 0.940] | 4.73 | False |
| mango | dry_matter | S4 | 30 | TJ+ | 0.900 [0.894, 0.906] | 4.4 | False |
| mango | dry_matter | S4 | 30 | TJ+N | 0.900 [0.894, 0.906] | 4.34 | False |
| mango | dry_matter | S4 | 30 | WCP-S | 0.743 [0.741, 0.745] | ∞ | True |
| mango | dry_matter | S4 | 50 | ASTM-PI | 0.762 [0.761, 0.764] | 4.7 | True |
| mango | dry_matter | S4 | 50 | Gauss-LOO | 0.908 [0.904, 0.913] | 4.26 | False |
| mango | dry_matter | S4 | 50 | SCP-S | 0.743 [0.742, 0.744] | 4.84 | True |
| mango | dry_matter | S4 | 50 | SCP-T-split-N | 0.929 [0.923, 0.934] | 4.38 | False |
| mango | dry_matter | S4 | 50 | TJ+ | 0.903 [0.899, 0.908] | 4.26 | False |
| mango | dry_matter | S4 | 50 | TJ+N | 0.904 [0.899, 0.908] | 4.2 | False |
| mango | dry_matter | S4 | 50 | WCP-S | 0.750 [0.749, 0.751] | ∞ | True |
| tablets | assay | 2 | 10 | ASTM-PI | 0.846 [0.840, 0.852] | 27.3 | True |
| tablets | assay | 2 | 10 | Gauss-LOO | 0.912 [0.904, 0.919] | 31.2 | False |
| tablets | assay | 2 | 10 | SCP-S | 0.814 [0.807, 0.819] | 28.8 | True |
| tablets | assay | 2 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| tablets | assay | 2 | 10 | TJ+ | 0.945 [0.940, 0.951] | 33.1 | False |
| tablets | assay | 2 | 10 | TJ+N | 0.946 [0.940, 0.950] | 31.2 | False |
| tablets | assay | 2 | 10 | Transport | 0.975 [0.973, 0.977] | 35.8 | False |
| tablets | assay | 2 | 10 | WCP-S | 0.853 [0.846, 0.859] | ∞ | True |
| tablets | assay | 2 | 20 | ASTM-PI | 0.927 [0.924, 0.930] | 21.6 | False |
| tablets | assay | 2 | 20 | Gauss-LOO | 0.915 [0.910, 0.920] | 23.1 | False |
| tablets | assay | 2 | 20 | SCP-S | 0.909 [0.907, 0.912] | 21.7 | False |
| tablets | assay | 2 | 20 | SCP-T-split-N | 0.914 [0.906, 0.920] | 28.2 | False |
| tablets | assay | 2 | 20 | TJ+ | 0.917 [0.912, 0.922] | 23.3 | False |
| tablets | assay | 2 | 20 | TJ+N | 0.918 [0.913, 0.923] | 21.9 | False |
| tablets | assay | 2 | 20 | Transport | 0.950 [0.947, 0.952] | 23.8 | False |
| tablets | assay | 2 | 20 | WCP-S | 0.926 [0.923, 0.929] | 23.5 | False |
| tablets | assay | 2 | 30 | ASTM-PI | 0.941 [0.940, 0.943] | 21.1 | False |
| tablets | assay | 2 | 30 | Gauss-LOO | 0.914 [0.909, 0.919] | 22.1 | False |
| tablets | assay | 2 | 30 | SCP-S | 0.926 [0.924, 0.928] | 21 | False |
| tablets | assay | 2 | 30 | SCP-T-split-N | 0.933 [0.927, 0.939] | 24.1 | False |
| tablets | assay | 2 | 30 | TJ+ | 0.905 [0.900, 0.910] | 21.7 | False |
| tablets | assay | 2 | 30 | TJ+N | 0.904 [0.899, 0.909] | 20.4 | False |
| tablets | assay | 2 | 30 | Transport | 0.947 [0.945, 0.949] | 22.2 | False |
| tablets | assay | 2 | 30 | WCP-S | 0.939 [0.937, 0.941] | 21.8 | False |
| tablets | assay | 2 | 50 | ASTM-PI | 0.952 [0.951, 0.953] | 20.8 | False |
| tablets | assay | 2 | 50 | Gauss-LOO | 0.926 [0.922, 0.929] | 21.1 | False |
| tablets | assay | 2 | 50 | SCP-S | 0.938 [0.936, 0.939] | 20.5 | False |
| tablets | assay | 2 | 50 | SCP-T-split-N | 0.922 [0.917, 0.927] | 21.1 | False |
| tablets | assay | 2 | 50 | TJ+ | 0.906 [0.902, 0.910] | 20.5 | False |
| tablets | assay | 2 | 50 | TJ+N | 0.905 [0.901, 0.909] | 19.4 | False |
| tablets | assay | 2 | 50 | Transport | 0.950 [0.949, 0.952] | 21.3 | False |
| tablets | assay | 2 | 50 | WCP-S | 0.949 [0.947, 0.950] | 21.1 | False |
| tablets | hardness | 2 | 10 | ASTM-PI | 0.949 [0.949, 0.950] | 4.75 | False |
| tablets | hardness | 2 | 10 | Gauss-LOO | 0.895 [0.888, 0.902] | 4.75 | False |
| tablets | hardness | 2 | 10 | SCP-S | 0.938 [0.937, 0.939] | 4.66 | False |
| tablets | hardness | 2 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| tablets | hardness | 2 | 10 | TJ+ | 0.910 [0.903, 0.917] | 4.86 | False |
| tablets | hardness | 2 | 10 | TJ+N | 0.911 [0.904, 0.918] | 4.46 | False |
| tablets | hardness | 2 | 10 | Transport | 0.938 [0.938, 0.939] | 4.67 | False |
| tablets | hardness | 2 | 10 | WCP-S | 0.938 [0.938, 0.939] | 4.66 | False |
| tablets | hardness | 2 | 20 | ASTM-PI | 0.949 [0.949, 0.949] | 4.77 | False |
| tablets | hardness | 2 | 20 | Gauss-LOO | 0.906 [0.901, 0.911] | 4.66 | False |
| tablets | hardness | 2 | 20 | SCP-S | 0.939 [0.938, 0.939] | 4.68 | False |
| tablets | hardness | 2 | 20 | SCP-T-split-N | 0.910 [0.902, 0.918] | 4.49 | False |
| tablets | hardness | 2 | 20 | TJ+ | 0.910 [0.904, 0.915] | 4.74 | False |
| tablets | hardness | 2 | 20 | TJ+N | 0.910 [0.904, 0.916] | 4.35 | False |
| tablets | hardness | 2 | 20 | Transport | 0.939 [0.939, 0.939] | 4.69 | False |
| tablets | hardness | 2 | 20 | WCP-S | 0.939 [0.938, 0.939] | 4.68 | False |
| tablets | hardness | 2 | 30 | ASTM-PI | 0.949 [0.948, 0.949] | 4.77 | False |
| tablets | hardness | 2 | 30 | Gauss-LOO | 0.906 [0.902, 0.910] | 4.64 | False |
| tablets | hardness | 2 | 30 | SCP-S | 0.939 [0.938, 0.939] | 4.68 | False |
| tablets | hardness | 2 | 30 | SCP-T-split-N | 0.934 [0.928, 0.940] | 4.54 | False |
| tablets | hardness | 2 | 30 | TJ+ | 0.904 [0.899, 0.909] | 4.68 | False |
| tablets | hardness | 2 | 30 | TJ+N | 0.904 [0.899, 0.908] | 4.3 | False |
| tablets | hardness | 2 | 30 | Transport | 0.939 [0.939, 0.939] | 4.7 | False |
| tablets | hardness | 2 | 30 | WCP-S | 0.939 [0.938, 0.939] | 4.68 | False |
| tablets | hardness | 2 | 50 | ASTM-PI | 0.949 [0.948, 0.949] | 4.78 | False |
| tablets | hardness | 2 | 50 | Gauss-LOO | 0.907 [0.904, 0.911] | 4.61 | False |
| tablets | hardness | 2 | 50 | SCP-S | 0.939 [0.938, 0.939] | 4.69 | False |
| tablets | hardness | 2 | 50 | SCP-T-split-N | 0.919 [0.915, 0.924] | 4.38 | False |
| tablets | hardness | 2 | 50 | TJ+ | 0.901 [0.897, 0.905] | 4.64 | False |
| tablets | hardness | 2 | 50 | TJ+N | 0.900 [0.896, 0.903] | 4.28 | False |
| tablets | hardness | 2 | 50 | Transport | 0.939 [0.939, 0.939] | 4.7 | False |
| tablets | hardness | 2 | 50 | WCP-S | 0.939 [0.938, 0.939] | 4.69 | False |
| tablets | weight | 2 | 10 | ASTM-PI | 0.911 [0.911, 0.912] | 17 | False |
| tablets | weight | 2 | 10 | Gauss-LOO | 0.907 [0.899, 0.914] | 18 | False |
| tablets | weight | 2 | 10 | SCP-S | 0.895 [0.894, 0.896] | 17 | True |
| tablets | weight | 2 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | False |
| tablets | weight | 2 | 10 | TJ+ | 0.914 [0.906, 0.921] | 18.4 | False |
| tablets | weight | 2 | 10 | TJ+N | 0.919 [0.913, 0.926] | 18.8 | False |
| tablets | weight | 2 | 10 | Transport | 0.887 [0.886, 0.888] | 17.1 | True |
| tablets | weight | 2 | 10 | WCP-S | 0.895 [0.894, 0.896] | 17 | True |
| tablets | weight | 2 | 20 | ASTM-PI | 0.910 [0.910, 0.911] | 17.1 | False |
| tablets | weight | 2 | 20 | Gauss-LOO | 0.905 [0.899, 0.910] | 17.6 | False |
| tablets | weight | 2 | 20 | SCP-S | 0.895 [0.894, 0.895] | 17.1 | True |
| tablets | weight | 2 | 20 | SCP-T-split-N | 0.914 [0.906, 0.921] | 19 | False |
| tablets | weight | 2 | 20 | TJ+ | 0.910 [0.904, 0.915] | 17.6 | False |
| tablets | weight | 2 | 20 | TJ+N | 0.906 [0.901, 0.912] | 18.3 | False |
| tablets | weight | 2 | 20 | Transport | 0.885 [0.884, 0.886] | 17.2 | True |
| tablets | weight | 2 | 20 | WCP-S | 0.895 [0.894, 0.895] | 17.1 | True |
| tablets | weight | 2 | 30 | ASTM-PI | 0.909 [0.909, 0.910] | 17.2 | False |
| tablets | weight | 2 | 30 | Gauss-LOO | 0.896 [0.892, 0.901] | 17.5 | False |
| tablets | weight | 2 | 30 | SCP-S | 0.894 [0.894, 0.895] | 17.2 | True |
| tablets | weight | 2 | 30 | SCP-T-split-N | 0.932 [0.927, 0.938] | 19.4 | False |
| tablets | weight | 2 | 30 | TJ+ | 0.900 [0.895, 0.905] | 17.5 | False |
| tablets | weight | 2 | 30 | TJ+N | 0.898 [0.893, 0.903] | 18.1 | False |
| tablets | weight | 2 | 30 | Transport | 0.884 [0.883, 0.885] | 17.2 | True |
| tablets | weight | 2 | 30 | WCP-S | 0.894 [0.894, 0.895] | 17.2 | True |
| tablets | weight | 2 | 50 | ASTM-PI | 0.909 [0.909, 0.910] | 17.1 | False |
| tablets | weight | 2 | 50 | Gauss-LOO | 0.907 [0.904, 0.911] | 17.4 | False |
| tablets | weight | 2 | 50 | SCP-S | 0.895 [0.894, 0.895] | 17.2 | True |
| tablets | weight | 2 | 50 | SCP-T-split-N | 0.926 [0.922, 0.930] | 18.5 | False |
| tablets | weight | 2 | 50 | TJ+ | 0.906 [0.902, 0.909] | 17.4 | False |
| tablets | weight | 2 | 50 | TJ+N | 0.905 [0.901, 0.908] | 18.1 | False |
| tablets | weight | 2 | 50 | Transport | 0.883 [0.883, 0.884] | 17.2 | True |
| tablets | weight | 2 | 50 | WCP-S | 0.895 [0.894, 0.895] | 17.2 | True |

# Table S2. All transfer pipelines

Table S2. Coverage (mean and minimum over scenarios), median interval score, RMSEP and under-covering scenarios for every transfer pipeline (α = 0.10).

| dataset | kind | method | coverage | coverage_min | IS_median | rmsep | n_under | n_scen |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| corn | ds | ASTM-PI | 0.131 | 0.035 | 3.884 | 0.392 | 24 | 24 |
| corn | ds | Gauss-LOO | 0.897 | 0.884 | 1.354 | 0.392 | 4 | 24 |
| corn | ds | SCP-S | 0.294 | 0.066 | 3.534 | 0.392 | 24 | 24 |
| corn | ds | SCP-T-split-N | 0.969 | 0.896 | inf | 0.392 | 0 | 24 |
| corn | ds | TJ+ | 0.932 | 0.91 | 1.435 | 0.392 | 0 | 24 |
| corn | ds | TJ+N | 0.93 | 0.907 | 1.472 | 0.392 | 0 | 24 |
| corn | none | ASTM-PI | 0.024 | 0.0 | 21.598 | 1.144 | 24 | 24 |
| corn | none | Gauss-LOO | 0.992 | 0.98 | 4.181 | 1.144 | 0 | 24 |
| corn | none | SCP-S | 0.041 | 0.0 | 21.556 | 1.144 | 24 | 24 |
| corn | none | SCP-T-split-N | 0.969 | 0.897 | inf | 1.144 | 0 | 24 |
| corn | none | TJ+ | 0.916 | 0.898 | 3.127 | 1.144 | 0 | 24 |
| corn | none | TJ+N | 0.916 | 0.899 | 3.149 | 1.144 | 0 | 24 |
| corn | pds | ASTM-PI | 0.233 | 0.026 | 3.045 | 0.292 | 24 | 24 |
| corn | pds | Gauss-LOO | 0.904 | 0.893 | 1.055 | 0.292 | 0 | 24 |
| corn | pds | SCP-S | 0.453 | 0.051 | 1.809 | 0.292 | 24 | 24 |
| corn | pds | SCP-T-split-N | 0.97 | 0.901 | inf | 0.292 | 0 | 24 |
| corn | pds | TJ+ | 0.933 | 0.908 | 1.107 | 0.292 | 0 | 24 |
| corn | pds | TJ+N | 0.931 | 0.908 | 1.138 | 0.292 | 0 | 24 |
| corn | sbc | ASTM-PI | 0.409 | 0.136 | 1.743 | 0.221 | 24 | 24 |
| corn | sbc | Gauss-LOO | 0.9 | 0.887 | 0.849 | 0.221 | 2 | 24 |
| corn | sbc | SCP-S | 0.515 | 0.073 | 1.424 | 0.221 | 24 | 24 |
| corn | sbc | SCP-T-split-N | 0.971 | 0.907 | inf | 0.221 | 0 | 24 |
| corn | sbc | TJ+ | 0.917 | 0.899 | 0.868 | 0.221 | 0 | 24 |
| corn | sbc | TJ+N | 0.918 | 0.902 | 0.863 | 0.221 | 0 | 24 |
| mango | none | ASTM-PI | 0.753 | 0.753 | 4.762 | 1.052 | 4 | 4 |
| mango | none | Gauss-LOO | 0.905 | 0.901 | 4.282 | 1.052 | 0 | 4 |
| mango | none | SCP-S | 0.735 | 0.735 | 4.915 | 1.052 | 4 | 4 |
| mango | none | SCP-T-split-N | 0.941 | 0.903 | 4.333 | 1.052 | 0 | 4 |
| mango | none | TJ+ | 0.904 | 0.897 | 4.29 | 1.052 | 0 | 4 |
| mango | none | TJ+N | 0.905 | 0.901 | 4.176 | 1.052 | 0 | 4 |
| mango | sbc | ASTM-PI | 0.748 | 0.726 | 4.888 | 1.096 | 4 | 4 |
| mango | sbc | Gauss-LOO | 0.904 | 0.902 | 4.446 | 1.096 | 0 | 4 |
| mango | sbc | SCP-S | 0.729 | 0.707 | 5.034 | 1.096 | 4 | 4 |
| mango | sbc | SCP-T-split-N | 0.942 | 0.905 | 4.807 | 1.096 | 0 | 4 |
| mango | sbc | TJ+ | 0.904 | 0.9 | 4.472 | 1.096 | 0 | 4 |
| mango | sbc | TJ+N | 0.904 | 0.9 | 4.398 | 1.096 | 0 | 4 |
| tablets | ds | ASTM-PI | 0.897 | 0.658 | 16.999 | 3.935 | 3 | 12 |
| tablets | ds | Gauss-LOO | 0.908 | 0.897 | 17.387 | 3.935 | 0 | 12 |
| tablets | ds | SCP-S | 0.88 | 0.627 | 16.981 | 3.935 | 7 | 12 |
| tablets | ds | SCP-T-split-N | 0.942 | 0.908 | 22.316 | 3.935 | 0 | 12 |
| tablets | ds | TJ+ | 0.91 | 0.9 | 17.416 | 3.935 | 0 | 12 |
| tablets | ds | TJ+N | 0.909 | 0.898 | 18.232 | 3.935 | 0 | 12 |
| tablets | none | ASTM-PI | 0.905 | 0.865 | 18.313 | 4.285 | 8 | 12 |
| tablets | none | Gauss-LOO | 0.902 | 0.875 | 18.554 | 4.285 | 4 | 12 |
| tablets | none | SCP-S | 0.898 | 0.863 | 18.366 | 4.285 | 8 | 12 |
| tablets | none | SCP-T-split-N | 0.943 | 0.909 | 29.065 | 4.285 | 0 | 12 |
| tablets | none | TJ+ | 0.906 | 0.9 | 18.644 | 4.285 | 0 | 12 |
| tablets | none | TJ+N | 0.906 | 0.9 | 18.999 | 4.285 | 0 | 12 |
| tablets | pds | ASTM-PI | 0.925 | 0.846 | 17.138 | 3.492 | 1 | 12 |
| tablets | pds | Gauss-LOO | 0.908 | 0.895 | 17.508 | 3.492 | 0 | 12 |
| tablets | pds | SCP-S | 0.91 | 0.814 | 17.152 | 3.492 | 5 | 12 |
| tablets | pds | SCP-T-split-N | 0.942 | 0.91 | 20.254 | 3.492 | 0 | 12 |
| tablets | pds | TJ+ | 0.911 | 0.9 | 17.532 | 3.492 | 0 | 12 |
| tablets | pds | TJ+N | 0.91 | 0.898 | 18.224 | 3.492 | 0 | 12 |
| tablets | sbc | ASTM-PI | 0.904 | 0.798 | 15.911 | 4.084 | 4 | 12 |
| tablets | sbc | Gauss-LOO | 0.899 | 0.879 | 16.176 | 4.084 | 2 | 12 |
| tablets | sbc | SCP-S | 0.891 | 0.78 | 15.747 | 4.084 | 5 | 12 |
| tablets | sbc | SCP-T-split-N | 0.943 | 0.907 | 25.828 | 4.084 | 0 | 12 |
| tablets | sbc | TJ+ | 0.906 | 0.899 | 16.264 | 4.084 | 0 | 12 |
| tablets | sbc | TJ+N | 0.906 | 0.9 | 16.462 | 4.084 | 0 | 12 |

# Table S3. Sensitivity to transfer and model settings

Table S3. Sensitivity to the PDS half-window (hw1 = 3 channels, hw3 = 7, hw5 = 11; the primary analysis used 5 channels) and to the number of latent variables (lv ±2).

| dataset | setting | method | coverage | coverage_min | IS_median | rmsep | n_under | n_scen |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| corn | hw1 | ASTM-PI | 0.25 | 0.033 | 2.655 | 0.237 | 4 | 4 |
| corn | hw1 | Gauss-LOO | 0.903 | 0.89 | 0.932 | 0.237 | 0 | 4 |
| corn | hw1 | SCP-S | 0.496 | 0.064 | 1.566 | 0.237 | 4 | 4 |
| corn | hw1 | SCP-T-split-N | 1.0 | 1.0 | inf | 0.237 | 0 | 4 |
| corn | hw1 | TJ+ | 0.941 | 0.93 | 1.057 | 0.237 | 0 | 4 |
| corn | hw1 | TJ+N | 0.941 | 0.927 | 1.083 | 0.237 | 0 | 4 |
| corn | hw1 | WCP-S | 0.588 | 0.217 | inf | 0.237 | 4 | 4 |
| corn | hw3 | ASTM-PI | 0.213 | 0.028 | 3.314 | 0.348 | 4 | 4 |
| corn | hw3 | Gauss-LOO | 0.909 | 0.896 | 1.21 | 0.348 | 0 | 4 |
| corn | hw3 | SCP-S | 0.404 | 0.056 | 2.363 | 0.348 | 4 | 4 |
| corn | hw3 | SCP-T-split-N | 1.0 | 1.0 | inf | 0.348 | 0 | 4 |
| corn | hw3 | TJ+ | 0.962 | 0.951 | 1.313 | 0.348 | 0 | 4 |
| corn | hw3 | TJ+N | 0.954 | 0.938 | 1.272 | 0.348 | 0 | 4 |
| corn | hw3 | WCP-S | 0.509 | 0.217 | inf | 0.348 | 4 | 4 |
| corn | hw5 | ASTM-PI | 0.17 | 0.032 | 4.362 | 0.503 | 4 | 4 |
| corn | hw5 | Gauss-LOO | 0.903 | 0.896 | 1.549 | 0.503 | 0 | 4 |
| corn | hw5 | SCP-S | 0.314 | 0.053 | 4.002 | 0.503 | 4 | 4 |
| corn | hw5 | SCP-T-split-N | 1.0 | 1.0 | inf | 0.503 | 0 | 4 |
| corn | hw5 | TJ+ | 0.967 | 0.955 | 1.735 | 0.503 | 0 | 4 |
| corn | hw5 | TJ+N | 0.962 | 0.95 | 1.512 | 0.503 | 0 | 4 |
| corn | hw5 | WCP-S | 0.44 | 0.233 | inf | 0.503 | 4 | 4 |
| corn | lv+2 | ASTM-PI | 0.189 | 0.033 | 3.136 | 0.283 | 4 | 4 |
| corn | lv+2 | Gauss-LOO | 0.905 | 0.893 | 1.047 | 0.283 | 0 | 4 |
| corn | lv+2 | SCP-S | 0.436 | 0.059 | 1.872 | 0.283 | 4 | 4 |
| corn | lv+2 | SCP-T-split-N | 1.0 | 1.0 | inf | 0.283 | 0 | 4 |
| corn | lv+2 | TJ+ | 0.952 | 0.943 | 1.18 | 0.283 | 0 | 4 |
| corn | lv+2 | TJ+N | 0.949 | 0.939 | 1.2 | 0.283 | 0 | 4 |
| corn | lv+2 | WCP-S | 0.552 | 0.237 | inf | 0.283 | 4 | 4 |
| corn | lv-2 | ASTM-PI | 0.313 | 0.051 | 2.488 | 0.275 | 4 | 4 |
| corn | lv-2 | Gauss-LOO | 0.904 | 0.894 | 1.06 | 0.275 | 0 | 4 |
| corn | lv-2 | SCP-S | 0.516 | 0.083 | 1.616 | 0.275 | 4 | 4 |
| corn | lv-2 | SCP-T-split-N | 1.0 | 1.0 | inf | 0.275 | 0 | 4 |
| corn | lv-2 | TJ+ | 0.951 | 0.943 | 1.161 | 0.275 | 0 | 4 |
| corn | lv-2 | TJ+N | 0.944 | 0.933 | 1.191 | 0.275 | 0 | 4 |
| corn | lv-2 | WCP-S | 0.589 | 0.203 | 2.741 | 0.275 | 4 | 4 |
| mango | lv+2 | ASTM-PI | 0.737 | 0.737 | 5.054 | 1.108 | 1 | 1 |
| mango | lv+2 | Gauss-LOO | 0.898 | 0.898 | 4.538 | 1.108 | 0 | 1 |
| mango | lv+2 | SCP-S | 0.721 | 0.721 | 5.186 | 1.108 | 1 | 1 |
| mango | lv+2 | SCP-T-split-N | 0.902 | 0.902 | 4.989 | 1.108 | 0 | 1 |
| mango | lv+2 | TJ+ | 0.899 | 0.899 | 4.612 | 1.108 | 0 | 1 |
| mango | lv+2 | TJ+N | 0.898 | 0.898 | 4.535 | 1.108 | 0 | 1 |
| mango | lv+2 | WCP-S | 0.734 | 0.734 | inf | 1.108 | 1 | 1 |
| mango | lv-2 | ASTM-PI | 0.757 | 0.757 | 5.016 | 1.106 | 1 | 1 |
| mango | lv-2 | Gauss-LOO | 0.896 | 0.896 | 4.587 | 1.106 | 0 | 1 |
| mango | lv-2 | SCP-S | 0.745 | 0.745 | 5.117 | 1.106 | 1 | 1 |
| mango | lv-2 | SCP-T-split-N | 0.906 | 0.906 | 4.968 | 1.106 | 0 | 1 |
| mango | lv-2 | TJ+ | 0.9 | 0.9 | 4.642 | 1.106 | 0 | 1 |
| mango | lv-2 | TJ+N | 0.899 | 0.899 | 4.544 | 1.106 | 0 | 1 |
| mango | lv-2 | WCP-S | 0.793 | 0.793 | inf | 1.106 | 1 | 1 |
| tablets | hw1 | ASTM-PI | 0.943 | 0.943 | 20.805 | 4.767 | 0 | 1 |
| tablets | hw1 | Gauss-LOO | 0.911 | 0.911 | 21.754 | 4.767 | 0 | 1 |
| tablets | hw1 | SCP-S | 0.928 | 0.928 | 20.589 | 4.767 | 0 | 1 |
| tablets | hw1 | SCP-T-split-N | 0.912 | 0.912 | 23.102 | 4.767 | 0 | 1 |
| tablets | hw1 | TJ+ | 0.913 | 0.913 | 21.903 | 4.767 | 0 | 1 |
| tablets | hw1 | TJ+N | 0.916 | 0.916 | 20.449 | 4.767 | 0 | 1 |
| tablets | hw1 | WCP-S | 0.939 | 0.939 | 22.007 | 4.767 | 0 | 1 |
| tablets | hw3 | ASTM-PI | 0.906 | 0.906 | 22.434 | 5.39 | 0 | 1 |
| tablets | hw3 | Gauss-LOO | 0.91 | 0.91 | 24.192 | 5.39 | 0 | 1 |
| tablets | hw3 | SCP-S | 0.884 | 0.884 | 22.73 | 5.39 | 1 | 1 |
| tablets | hw3 | SCP-T-split-N | 0.92 | 0.92 | 33.539 | 5.39 | 0 | 1 |
| tablets | hw3 | TJ+ | 0.916 | 0.916 | 24.22 | 5.39 | 0 | 1 |
| tablets | hw3 | TJ+N | 0.916 | 0.916 | 23.243 | 5.39 | 0 | 1 |
| tablets | hw3 | WCP-S | 0.909 | 0.909 | 24.208 | 5.39 | 0 | 1 |
| tablets | hw5 | ASTM-PI | 0.867 | 0.867 | 25.273 | 5.98 | 1 | 1 |
| tablets | hw5 | Gauss-LOO | 0.912 | 0.912 | 27.218 | 5.98 | 0 | 1 |
| tablets | hw5 | SCP-S | 0.837 | 0.837 | 26.267 | 5.98 | 1 | 1 |
| tablets | hw5 | SCP-T-split-N | 0.915 | 0.915 | 37.461 | 5.98 | 0 | 1 |
| tablets | hw5 | TJ+ | 0.925 | 0.925 | 27.284 | 5.98 | 0 | 1 |
| tablets | hw5 | TJ+N | 0.924 | 0.924 | 26.505 | 5.98 | 0 | 1 |
| tablets | hw5 | WCP-S | 0.87 | 0.87 | 29.995 | 5.98 | 1 | 1 |
| tablets | lv+2 | ASTM-PI | 0.898 | 0.898 | 22.363 | 5.326 | 0 | 1 |
| tablets | lv+2 | Gauss-LOO | 0.908 | 0.908 | 24.135 | 5.326 | 0 | 1 |
| tablets | lv+2 | SCP-S | 0.876 | 0.876 | 23.175 | 5.326 | 1 | 1 |
| tablets | lv+2 | SCP-T-split-N | 0.918 | 0.918 | 26.722 | 5.326 | 0 | 1 |
| tablets | lv+2 | TJ+ | 0.91 | 0.91 | 24.74 | 5.326 | 0 | 1 |
| tablets | lv+2 | TJ+N | 0.918 | 0.918 | 21.987 | 5.326 | 0 | 1 |
| tablets | lv+2 | WCP-S | 0.907 | 0.907 | inf | 5.326 | 0 | 1 |
| tablets | lv-2 | ASTM-PI | 0.903 | 0.903 | 29.591 | 6.999 | 0 | 1 |
| tablets | lv-2 | Gauss-LOO | 0.909 | 0.909 | 31.299 | 6.999 | 0 | 1 |
| tablets | lv-2 | SCP-S | 0.912 | 0.912 | 29.915 | 6.999 | 0 | 1 |
| tablets | lv-2 | SCP-T-split-N | 0.908 | 0.908 | 37.834 | 6.999 | 0 | 1 |
| tablets | lv-2 | TJ+ | 0.912 | 0.912 | 32.295 | 6.999 | 0 | 1 |
| tablets | lv-2 | TJ+N | 0.916 | 0.916 | 28.684 | 6.999 | 0 | 1 |
| tablets | lv-2 | WCP-S | 0.92 | 0.92 | 30.799 | 6.999 | 0 | 1 |

# Table S4. Computational cost

Table S4. Wall-clock time and peak memory (single CPU core, median of 20 runs).

| dataset | kind | m | n_test | n_channels | step | median_s | p90_s | peak_mem_MB | source_model_fit_s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| corn | pds | 20 | 10 | 700 | point prediction only (1 transfer fit) | 0.0046 | 0.0061 | 0.8785 | 0.1027 |
| corn | pds | 20 | 10 | 700 | SCP-T-split (1 fit) | 0.0047 | 0.0065 | 0.7389 | 0.1027 |
| corn | pds | 20 | 10 | 700 | TJ+N incl. m LOO + 5 CV fits | 0.1469 | 0.1572 | 1.2104 | 0.1027 |
| tablets | pds | 50 | 410 | 650 | point prediction only (1 transfer fit) | 0.0243 | 0.0298 | 14.3825 | 0.2743 |
| tablets | pds | 50 | 410 | 650 | SCP-T-split (1 fit) | 0.0259 | 0.0265 | 14.3831 | 0.2743 |
| tablets | pds | 50 | 410 | 650 | TJ+N incl. m LOO + 5 CV fits | 2.7857 | 2.9018 | 15.2063 | 0.2743 |
| mango | sbc | 50 | 675 | 103 | point prediction only (1 transfer fit) | 0.0004 | 0.0005 | 1.1246 | 1.4279 |
| mango | sbc | 50 | 675 | 103 | SCP-T-split (1 fit) | 0.0004 | 0.0004 | 1.1246 | 1.4279 |
| mango | sbc | 50 | 675 | 103 | TJ+N incl. m LOO + 5 CV fits | 0.0635 | 0.0742 | 3.1352 | 1.4279 |

# Table S5. Paired tests

Table S5. Paired Wilcoxon signed-rank tests of per-repetition interval score, TJ+N against each method: number of scenarios with a significantly lower (better) or higher (worse) score for TJ+N (Holm-adjusted p < 0.05 within scenario) and median matched-pairs rank-biserial correlation (negative = TJ+N better).

| dataset | other | n_scen | n_sig_TJ_better | median_rank_biserial | n_sig_TJ_worse |
| --- | --- | --- | --- | --- | --- |
| corn | ASTM-PI | 24 | 24 | -0.978 | 0 |
| corn | Gauss-LOO | 24 | 4 | 0.143 | 14 |
| corn | LF-conv | 24 | 4 | 0.056 | 8 |
| corn | SCP-S | 24 | 24 | -0.681 | 0 |
| corn | SCP-T-split | 24 | 8 | -1.0 | 0 |
| corn | SCP-T-split-N | 24 | 8 | -1.0 | 0 |
| corn | TCV+N | 24 | 24 | -0.417 | 0 |
| corn | TJ+ | 24 | 12 | -0.157 | 1 |
| corn | TJ+N[Q] | 24 | 4 | -0.335 | 0 |
| corn | TJ+N[T2] | 24 | 0 | 0.172 | 6 |
| corn | TJ-insample-N | 24 | 0 | 0.559 | 24 |
| corn | TJ-naive-N | 24 | 0 | 0.452 | 23 |
| corn | Transport | 24 | 20 | -0.33 | 0 |
| corn | WCP-S | 24 | 21 | -0.918 | 3 |
| mango | ASTM-PI | 4 | 4 | -0.929 | 0 |
| mango | Gauss-LOO | 4 | 2 | -0.26 | 1 |
| mango | SCP-S | 4 | 4 | -0.965 | 0 |
| mango | SCP-T-split | 4 | 3 | -0.861 | 0 |
| mango | SCP-T-split-N | 4 | 3 | -0.771 | 0 |
| mango | TCV+N | 4 | 4 | -0.186 | 0 |
| mango | TJ+ | 4 | 4 | -0.54 | 0 |
| mango | TJ+N[Q] | 4 | 4 | -0.5 | 0 |
| mango | TJ+N[T2] | 4 | 1 | -0.132 | 0 |
| mango | TJ-insample-N | 4 | 0 | 0.065 | 1 |
| mango | TJ-naive-N | 4 | 0 | 0.207 | 3 |
| mango | WCP-S | 4 | 0 | -1.0 | 0 |
| tablets | ASTM-PI | 12 | 7 | -0.18 | 5 |
| tablets | Gauss-LOO | 12 | 7 | -0.55 | 4 |
| tablets | LF-conv | 12 | 7 | -0.111 | 4 |
| tablets | SCP-S | 12 | 7 | -0.144 | 5 |
| tablets | SCP-T-split | 12 | 6 | -0.977 | 2 |
| tablets | SCP-T-split-N | 12 | 9 | -0.799 | 0 |
| tablets | TCV+N | 12 | 4 | 0.038 | 3 |
| tablets | TJ+ | 12 | 8 | -0.816 | 4 |
| tablets | TJ+N[Q] | 12 | 8 | -0.636 | 4 |
| tablets | TJ+N[T2] | 12 | 5 | 0.127 | 6 |
| tablets | TJ-insample-N | 12 | 7 | -0.246 | 3 |
| tablets | TJ-naive-N | 12 | 2 | 0.172 | 7 |
| tablets | Transport | 12 | 8 | -0.535 | 4 |
| tablets | WCP-S | 12 | 7 | -0.624 | 5 |

# Table S6. Independent instrument shift (mango, E8)

Table S6. Mango instrument transfer (source instrument 15006; α = 0.10; 200 repetitions).

| Pipeline | Target instrument | m | Method | Coverage [95% CI] | Median IS | RMSEP | Under-covering |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pds | 15014 | 10 | ASTM-PI | 0.295 [0.270, 0.318] | 44.2 | 5.594 | True |
| pds | 15014 | 10 | Gauss-LOO | 0.928 [0.916, 0.940] | 22 | 5.594 | False |
| pds | 15014 | 10 | LF-conv | 0.878 [0.861, 0.894] | 23.6 | 5.594 | True |
| pds | 15014 | 10 | SCP-S | 0.280 [0.256, 0.304] | 46 | 5.594 | True |
| pds | 15014 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | 5.594 | False |
| pds | 15014 | 10 | TJ+ | 0.972 [0.965, 0.979] | 27.7 | 5.594 | False |
| pds | 15014 | 10 | TJ+N | 0.973 [0.968, 0.978] | 23.6 | 5.594 | False |
| pds | 15014 | 10 | Transport | 0.974 [0.964, 0.983] | 37.6 | 5.594 | False |
| pds | 15014 | 10 | WCP-S | 0.277 [0.254, 0.300] | ∞ | 5.594 | True |
| sbc | 15014 | 10 | ASTM-PI | 0.738 [0.732, 0.744] | 5.79 | 1.2 | True |
| sbc | 15014 | 10 | Gauss-LOO | 0.899 [0.887, 0.910] | 5.37 | 1.2 | False |
| sbc | 15014 | 10 | SCP-S | 0.726 [0.720, 0.732] | 5.9 | 1.2 | True |
| sbc | 15014 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | 1.2 | False |
| sbc | 15014 | 10 | TJ+ | 0.915 [0.905, 0.926] | 5.6 | 1.2 | False |
| sbc | 15014 | 10 | TJ+N | 0.918 [0.906, 0.929] | 5.55 | 1.2 | False |
| sbc | 15014 | 10 | WCP-S | 0.999 [0.999, 0.999] | ∞ | 1.2 | False |
| pds | 15014 | 20 | ASTM-PI | 0.357 [0.334, 0.382] | 29 | 4.017 | True |
| pds | 15014 | 20 | Gauss-LOO | 0.914 [0.903, 0.925] | 14.8 | 4.017 | False |
| pds | 15014 | 20 | LF-conv | 0.894 [0.883, 0.905] | 15 | 4.017 | False |
| pds | 15014 | 20 | SCP-S | 0.345 [0.322, 0.370] | 30 | 4.017 | True |
| pds | 15014 | 20 | SCP-T-split-N | 0.913 [0.900, 0.924] | 14.6 | 4.017 | False |
| pds | 15014 | 20 | TJ+ | 0.933 [0.925, 0.940] | 14.8 | 4.017 | False |
| pds | 15014 | 20 | TJ+N | 0.941 [0.934, 0.947] | 11.9 | 4.017 | False |
| pds | 15014 | 20 | Transport | 0.958 [0.947, 0.968] | 21.9 | 4.017 | False |
| pds | 15014 | 20 | WCP-S | 0.309 [0.286, 0.331] | 52.2 | 4.017 | True |
| sbc | 15014 | 20 | ASTM-PI | 0.762 [0.759, 0.765] | 5.4 | 1.141 | True |
| sbc | 15014 | 20 | Gauss-LOO | 0.901 [0.893, 0.909] | 5.01 | 1.141 | False |
| sbc | 15014 | 20 | SCP-S | 0.751 [0.748, 0.754] | 5.5 | 1.141 | True |
| sbc | 15014 | 20 | SCP-T-split-N | 0.912 [0.901, 0.923] | 5.42 | 1.141 | False |
| sbc | 15014 | 20 | TJ+ | 0.910 [0.900, 0.919] | 5.13 | 1.141 | False |
| sbc | 15014 | 20 | TJ+N | 0.912 [0.903, 0.921] | 5.08 | 1.141 | False |
| sbc | 15014 | 20 | WCP-S | 1.000 [0.999, 1.000] | ∞ | 1.141 | False |
| pds | 15014 | 30 | ASTM-PI | 0.400 [0.375, 0.426] | 23.8 | 3.421 | True |
| pds | 15014 | 30 | Gauss-LOO | 0.912 [0.904, 0.920] | 12 | 3.421 | False |
| pds | 15014 | 30 | LF-conv | 0.888 [0.879, 0.897] | 12.2 | 3.421 | True |
| pds | 15014 | 30 | SCP-S | 0.389 [0.362, 0.416] | 24.4 | 3.421 | True |
| pds | 15014 | 30 | SCP-T-split-N | 0.939 [0.930, 0.946] | 12.1 | 3.421 | False |
| pds | 15014 | 30 | TJ+ | 0.922 [0.915, 0.928] | 11.9 | 3.421 | False |
| pds | 15014 | 30 | TJ+N | 0.924 [0.918, 0.929] | 10 | 3.421 | False |
| pds | 15014 | 30 | Transport | 0.955 [0.943, 0.965] | 16 | 3.421 | False |
| pds | 15014 | 30 | WCP-S | 0.356 [0.332, 0.380] | 35.3 | 3.421 | True |
| sbc | 15014 | 30 | ASTM-PI | 0.768 [0.765, 0.770] | 5.33 | 1.127 | True |
| sbc | 15014 | 30 | Gauss-LOO | 0.903 [0.897, 0.910] | 4.88 | 1.127 | False |
| sbc | 15014 | 30 | SCP-S | 0.756 [0.753, 0.759] | 5.42 | 1.127 | True |
| sbc | 15014 | 30 | SCP-T-split-N | 0.938 [0.929, 0.945] | 5.34 | 1.127 | False |
| sbc | 15014 | 30 | TJ+ | 0.906 [0.898, 0.913] | 4.93 | 1.127 | False |
| sbc | 15014 | 30 | TJ+N | 0.906 [0.898, 0.913] | 4.91 | 1.127 | False |
| sbc | 15014 | 30 | WCP-S | 1.000 [0.999, 1.000] | ∞ | 1.127 | False |
| pds | 15014 | 50 | ASTM-PI | 0.408 [0.380, 0.436] | 18.9 | 3.43 | True |
| pds | 15014 | 50 | Gauss-LOO | 0.904 [0.897, 0.911] | 9.91 | 3.43 | False |
| pds | 15014 | 50 | LF-conv | 0.885 [0.877, 0.892] | 10.1 | 3.43 | True |
| pds | 15014 | 50 | SCP-S | 0.397 [0.371, 0.425] | 19.4 | 3.43 | True |
| pds | 15014 | 50 | SCP-T-split-N | 0.920 [0.912, 0.927] | 10.7 | 3.43 | False |
| pds | 15014 | 50 | TJ+ | 0.907 [0.901, 0.912] | 10.1 | 3.43 | False |
| pds | 15014 | 50 | TJ+N | 0.909 [0.904, 0.915] | 8.42 | 3.43 | False |
| pds | 15014 | 50 | Transport | 0.945 [0.932, 0.958] | 15.1 | 3.43 | False |
| pds | 15014 | 50 | WCP-S | 0.356 [0.331, 0.381] | 21.8 | 3.43 | True |
| sbc | 15014 | 50 | ASTM-PI | 0.773 [0.772, 0.774] | 5.28 | 1.115 | True |
| sbc | 15014 | 50 | Gauss-LOO | 0.896 [0.891, 0.902] | 4.8 | 1.115 | False |
| sbc | 15014 | 50 | SCP-S | 0.762 [0.760, 0.763] | 5.38 | 1.115 | True |
| sbc | 15014 | 50 | SCP-T-split-N | 0.924 [0.917, 0.931] | 4.98 | 1.115 | False |
| sbc | 15014 | 50 | TJ+ | 0.899 [0.893, 0.905] | 4.84 | 1.115 | False |
| sbc | 15014 | 50 | TJ+N | 0.899 [0.893, 0.905] | 4.77 | 1.115 | False |
| sbc | 15014 | 50 | WCP-S | 1.000 [1.000, 1.000] | ∞ | 1.115 | False |
| pds | 15016 | 10 | ASTM-PI | 0.219 [0.203, 0.236] | 66.2 | 7.401 | True |
| pds | 15016 | 10 | Gauss-LOO | 0.922 [0.907, 0.935] | 29.5 | 7.401 | False |
| pds | 15016 | 10 | LF-conv | 0.859 [0.840, 0.878] | 31.5 | 7.401 | True |
| pds | 15016 | 10 | SCP-S | 0.204 [0.188, 0.220] | 68.9 | 7.401 | True |
| pds | 15016 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | 7.401 | False |
| pds | 15016 | 10 | TJ+ | 0.956 [0.945, 0.966] | 36.5 | 7.401 | False |
| pds | 15016 | 10 | TJ+N | 0.965 [0.956, 0.973] | 27.6 | 7.401 | False |
| pds | 15016 | 10 | Transport | 0.989 [0.984, 0.994] | 69.2 | 7.401 | False |
| pds | 15016 | 10 | WCP-S | 0.151 [0.139, 0.164] | 176 | 7.401 | True |
| sbc | 15016 | 10 | ASTM-PI | 0.767 [0.761, 0.772] | 5.55 | 1.224 | True |
| sbc | 15016 | 10 | Gauss-LOO | 0.894 [0.881, 0.907] | 5.33 | 1.224 | False |
| sbc | 15016 | 10 | SCP-S | 0.729 [0.723, 0.735] | 5.88 | 1.224 | True |
| sbc | 15016 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | 1.224 | False |
| sbc | 15016 | 10 | TJ+ | 0.902 [0.888, 0.917] | 5.64 | 1.224 | False |
| sbc | 15016 | 10 | TJ+N | 0.905 [0.891, 0.919] | 6.34 | 1.224 | False |
| sbc | 15016 | 10 | WCP-S | 1.000 [1.000, 1.000] | ∞ | 1.224 | False |
| pds | 15016 | 20 | ASTM-PI | 0.262 [0.246, 0.279] | 50 | 5.086 | True |
| pds | 15016 | 20 | Gauss-LOO | 0.921 [0.912, 0.930] | 21.4 | 5.086 | False |
| pds | 15016 | 20 | LF-conv | 0.906 [0.895, 0.915] | 21.5 | 5.086 | False |
| pds | 15016 | 20 | SCP-S | 0.248 [0.233, 0.263] | 51.8 | 5.086 | True |
| pds | 15016 | 20 | SCP-T-split-N | 0.923 [0.913, 0.933] | 22 | 5.086 | False |
| pds | 15016 | 20 | TJ+ | 0.929 [0.922, 0.937] | 21.8 | 5.086 | False |
| pds | 15016 | 20 | TJ+N | 0.942 [0.935, 0.948] | 17.1 | 5.086 | False |
| pds | 15016 | 20 | Transport | 0.991 [0.986, 0.995] | 44.1 | 5.086 | False |
| pds | 15016 | 20 | WCP-S | 0.159 [0.148, 0.170] | 74.3 | 5.086 | True |
| sbc | 15016 | 20 | ASTM-PI | 0.790 [0.786, 0.793] | 5.24 | 1.173 | True |
| sbc | 15016 | 20 | Gauss-LOO | 0.900 [0.891, 0.908] | 5 | 1.173 | False |
| sbc | 15016 | 20 | SCP-S | 0.753 [0.750, 0.756] | 5.49 | 1.173 | True |
| sbc | 15016 | 20 | SCP-T-split-N | 0.905 [0.894, 0.916] | 6.09 | 1.173 | False |
| sbc | 15016 | 20 | TJ+ | 0.905 [0.895, 0.914] | 5.08 | 1.173 | False |
| sbc | 15016 | 20 | TJ+N | 0.905 [0.896, 0.914] | 5.55 | 1.173 | False |
| sbc | 15016 | 20 | WCP-S | 1.000 [1.000, 1.000] | ∞ | 1.173 | False |
| pds | 15016 | 30 | ASTM-PI | 0.293 [0.279, 0.308] | 43.1 | 4.33 | True |
| pds | 15016 | 30 | Gauss-LOO | 0.916 [0.908, 0.923] | 18 | 4.33 | False |
| pds | 15016 | 30 | LF-conv | 0.897 [0.889, 0.905] | 18.5 | 4.33 | False |
| pds | 15016 | 30 | SCP-S | 0.280 [0.266, 0.294] | 44.5 | 4.33 | True |
| pds | 15016 | 30 | SCP-T-split-N | 0.938 [0.930, 0.946] | 17.9 | 4.33 | False |
| pds | 15016 | 30 | TJ+ | 0.918 [0.912, 0.925] | 18.1 | 4.33 | False |
| pds | 15016 | 30 | TJ+N | 0.915 [0.909, 0.922] | 14.2 | 4.33 | False |
| pds | 15016 | 30 | Transport | 0.984 [0.977, 0.989] | 32.7 | 4.33 | False |
| pds | 15016 | 30 | WCP-S | 0.168 [0.158, 0.178] | 54.4 | 4.33 | True |
| sbc | 15016 | 30 | ASTM-PI | 0.797 [0.794, 0.800] | 5.13 | 1.155 | True |
| sbc | 15016 | 30 | Gauss-LOO | 0.902 [0.895, 0.909] | 4.89 | 1.155 | False |
| sbc | 15016 | 30 | SCP-S | 0.761 [0.758, 0.763] | 5.36 | 1.155 | True |
| sbc | 15016 | 30 | SCP-T-split-N | 0.937 [0.928, 0.945] | 6.08 | 1.155 | False |
| sbc | 15016 | 30 | TJ+ | 0.900 [0.893, 0.908] | 4.93 | 1.155 | False |
| sbc | 15016 | 30 | TJ+N | 0.901 [0.894, 0.909] | 5.32 | 1.155 | False |
| sbc | 15016 | 30 | WCP-S | 1.000 [1.000, 1.000] | ∞ | 1.155 | False |
| pds | 15016 | 50 | ASTM-PI | 0.317 [0.304, 0.331] | 36.9 | 3.924 | True |
| pds | 15016 | 50 | Gauss-LOO | 0.911 [0.905, 0.916] | 16.5 | 3.924 | False |
| pds | 15016 | 50 | LF-conv | 0.896 [0.890, 0.902] | 16.4 | 3.924 | False |
| pds | 15016 | 50 | SCP-S | 0.304 [0.291, 0.317] | 38 | 3.924 | True |
| pds | 15016 | 50 | SCP-T-split-N | 0.921 [0.914, 0.928] | 14.6 | 3.924 | False |
| pds | 15016 | 50 | TJ+ | 0.903 [0.898, 0.909] | 16.2 | 3.924 | False |
| pds | 15016 | 50 | TJ+N | 0.907 [0.902, 0.913] | 12.6 | 3.924 | False |
| pds | 15016 | 50 | Transport | 0.984 [0.980, 0.988] | 26.4 | 3.924 | False |
| pds | 15016 | 50 | WCP-S | 0.175 [0.167, 0.184] | 46.1 | 3.924 | True |
| sbc | 15016 | 50 | ASTM-PI | 0.803 [0.801, 0.805] | 5.05 | 1.141 | True |
| sbc | 15016 | 50 | Gauss-LOO | 0.905 [0.900, 0.910] | 4.79 | 1.141 | False |
| sbc | 15016 | 50 | SCP-S | 0.766 [0.763, 0.767] | 5.28 | 1.141 | True |
| sbc | 15016 | 50 | SCP-T-split-N | 0.916 [0.908, 0.923] | 5.41 | 1.141 | False |
| sbc | 15016 | 50 | TJ+ | 0.902 [0.897, 0.908] | 4.83 | 1.141 | False |
| sbc | 15016 | 50 | TJ+N | 0.903 [0.897, 0.908] | 5.17 | 1.141 | False |
| sbc | 15016 | 50 | WCP-S | 1.000 [1.000, 1.000] | ∞ | 1.141 | False |
| pds | 15029 | 10 | ASTM-PI | 0.332 [0.311, 0.352] | 30.8 | 3.95 | True |
| pds | 15029 | 10 | Gauss-LOO | 0.923 [0.907, 0.937] | 16.6 | 3.95 | False |
| pds | 15029 | 10 | LF-conv | 0.866 [0.847, 0.884] | 16.9 | 3.95 | True |
| pds | 15029 | 10 | SCP-S | 0.320 [0.299, 0.340] | 31.6 | 3.95 | True |
| pds | 15029 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | 3.95 | False |
| pds | 15029 | 10 | TJ+ | 0.961 [0.953, 0.968] | 19.6 | 3.95 | False |
| pds | 15029 | 10 | TJ+N | 0.960 [0.953, 0.965] | 17.6 | 3.95 | False |
| pds | 15029 | 10 | Transport | 0.992 [0.988, 0.996] | 42.5 | 3.95 | False |
| pds | 15029 | 10 | WCP-S | 0.308 [0.286, 0.329] | ∞ | 3.95 | True |
| sbc | 15029 | 10 | ASTM-PI | 0.773 [0.765, 0.780] | 5.36 | 1.155 | True |
| sbc | 15029 | 10 | Gauss-LOO | 0.893 [0.880, 0.905] | 5.31 | 1.155 | False |
| sbc | 15029 | 10 | SCP-S | 0.763 [0.756, 0.770] | 5.42 | 1.155 | True |
| sbc | 15029 | 10 | SCP-T-split-N | 1.000 [1.000, 1.000] | ∞ | 1.155 | False |
| sbc | 15029 | 10 | TJ+ | 0.910 [0.898, 0.922] | 5.51 | 1.155 | False |
| sbc | 15029 | 10 | TJ+N | 0.910 [0.898, 0.922] | 5.66 | 1.155 | False |
| sbc | 15029 | 10 | WCP-S | 0.999 [0.999, 0.999] | ∞ | 1.155 | False |
| pds | 15029 | 20 | ASTM-PI | 0.408 [0.390, 0.426] | 20.4 | 2.779 | True |
| pds | 15029 | 20 | Gauss-LOO | 0.920 [0.910, 0.929] | 10.4 | 2.779 | False |
| pds | 15029 | 20 | LF-conv | 0.900 [0.889, 0.911] | 10.8 | 2.779 | False |
| pds | 15029 | 20 | SCP-S | 0.397 [0.379, 0.415] | 21 | 2.779 | True |
| pds | 15029 | 20 | SCP-T-split-N | 0.909 [0.898, 0.920] | 13 | 2.779 | False |
| pds | 15029 | 20 | TJ+ | 0.924 [0.915, 0.932] | 11 | 2.779 | False |
| pds | 15029 | 20 | TJ+N | 0.922 [0.914, 0.930] | 10.6 | 2.779 | False |
| pds | 15029 | 20 | Transport | 0.989 [0.984, 0.993] | 24.5 | 2.779 | False |
| pds | 15029 | 20 | WCP-S | 0.364 [0.344, 0.385] | 27.9 | 2.779 | True |
| sbc | 15029 | 20 | ASTM-PI | 0.796 [0.792, 0.799] | 5.05 | 1.091 | True |
| sbc | 15029 | 20 | Gauss-LOO | 0.898 [0.889, 0.906] | 4.8 | 1.091 | False |
| sbc | 15029 | 20 | SCP-S | 0.787 [0.783, 0.790] | 5.11 | 1.091 | True |
| sbc | 15029 | 20 | SCP-T-split-N | 0.899 [0.887, 0.911] | 5.41 | 1.091 | False |
| sbc | 15029 | 20 | TJ+ | 0.905 [0.895, 0.913] | 4.91 | 1.091 | False |
| sbc | 15029 | 20 | TJ+N | 0.906 [0.897, 0.915] | 4.99 | 1.091 | False |
| sbc | 15029 | 20 | WCP-S | 0.999 [0.999, 0.999] | ∞ | 1.091 | False |
| pds | 15029 | 30 | ASTM-PI | 0.458 [0.437, 0.479] | 16.1 | 2.398 | True |
| pds | 15029 | 30 | Gauss-LOO | 0.916 [0.908, 0.923] | 8.73 | 2.398 | False |
| pds | 15029 | 30 | LF-conv | 0.892 [0.883, 0.900] | 8.77 | 2.398 | False |
| pds | 15029 | 30 | SCP-S | 0.448 [0.428, 0.469] | 16.5 | 2.398 | True |
| pds | 15029 | 30 | SCP-T-split-N | 0.934 [0.925, 0.943] | 11.6 | 2.398 | False |
| pds | 15029 | 30 | TJ+ | 0.915 [0.908, 0.922] | 9.24 | 2.398 | False |
| pds | 15029 | 30 | TJ+N | 0.910 [0.904, 0.917] | 8.94 | 2.398 | False |
| pds | 15029 | 30 | Transport | 0.989 [0.985, 0.993] | 16.6 | 2.398 | False |
| pds | 15029 | 30 | WCP-S | 0.413 [0.392, 0.433] | 19.3 | 2.398 | True |
| sbc | 15029 | 30 | ASTM-PI | 0.802 [0.800, 0.805] | 4.92 | 1.07 | True |
| sbc | 15029 | 30 | Gauss-LOO | 0.897 [0.891, 0.903] | 4.7 | 1.07 | False |
| sbc | 15029 | 30 | SCP-S | 0.793 [0.790, 0.795] | 4.97 | 1.07 | True |
| sbc | 15029 | 30 | SCP-T-split-N | 0.935 [0.927, 0.943] | 5.23 | 1.07 | False |
| sbc | 15029 | 30 | TJ+ | 0.902 [0.895, 0.909] | 4.75 | 1.07 | False |
| sbc | 15029 | 30 | TJ+N | 0.902 [0.895, 0.909] | 4.84 | 1.07 | False |
| sbc | 15029 | 30 | WCP-S | 0.999 [0.999, 0.999] | ∞ | 1.07 | False |
| pds | 15029 | 50 | ASTM-PI | 0.507 [0.488, 0.525] | 12.4 | 2.032 | True |
| pds | 15029 | 50 | Gauss-LOO | 0.905 [0.900, 0.910] | 7.55 | 2.032 | False |
| pds | 15029 | 50 | LF-conv | 0.881 [0.876, 0.886] | 7.66 | 2.032 | True |
| pds | 15029 | 50 | SCP-S | 0.497 [0.479, 0.514] | 12.7 | 2.032 | True |
| pds | 15029 | 50 | SCP-T-split-N | 0.917 [0.909, 0.925] | 9.22 | 2.032 | False |
| pds | 15029 | 50 | TJ+ | 0.903 [0.898, 0.908] | 7.6 | 2.032 | False |
| pds | 15029 | 50 | TJ+N | 0.907 [0.901, 0.912] | 7.6 | 2.032 | False |
| pds | 15029 | 50 | Transport | 0.983 [0.978, 0.988] | 13.3 | 2.032 | False |
| pds | 15029 | 50 | WCP-S | 0.469 [0.449, 0.488] | 13.5 | 2.032 | True |
| sbc | 15029 | 50 | ASTM-PI | 0.806 [0.804, 0.808] | 4.84 | 1.057 | True |
| sbc | 15029 | 50 | Gauss-LOO | 0.906 [0.901, 0.911] | 4.58 | 1.057 | False |
| sbc | 15029 | 50 | SCP-S | 0.796 [0.794, 0.798] | 4.9 | 1.057 | True |
| sbc | 15029 | 50 | SCP-T-split-N | 0.923 [0.915, 0.930] | 4.93 | 1.057 | False |
| sbc | 15029 | 50 | TJ+ | 0.911 [0.906, 0.916] | 4.63 | 1.057 | False |
| sbc | 15029 | 50 | TJ+N | 0.910 [0.905, 0.915] | 4.69 | 1.057 | False |
| sbc | 15029 | 50 | WCP-S | 0.999 [0.999, 0.999] | ∞ | 1.057 | False |

# Table S7. Scaling of computational cost with m

Table S7. Wall-clock time of a single transfer fit with prediction and of TJ+N (including five CV+ refits) versus the number of standards m (single CPU core, median of 10 runs).

| dataset | kind | m | n_test | step | median_s |
| --- | --- | --- | --- | --- | --- |
| corn | pds | 10 | 20 | single transfer fit + prediction | 0.0047 |
| corn | pds | 10 | 20 | TJ+N (m LOO + 5 CV fits) | 0.101 |
| corn | pds | 20 | 10 | single transfer fit + prediction | 0.0044 |
| corn | pds | 20 | 10 | TJ+N (m LOO + 5 CV fits) | 0.146 |
| tablets | pds | 10 | 450 | single transfer fit + prediction | 0.0349 |
| tablets | pds | 10 | 450 | TJ+N (m LOO + 5 CV fits) | 0.8028 |
| tablets | pds | 20 | 440 | single transfer fit + prediction | 0.0239 |
| tablets | pds | 20 | 440 | TJ+N (m LOO + 5 CV fits) | 1.2477 |
| tablets | pds | 30 | 430 | single transfer fit + prediction | 0.0248 |
| tablets | pds | 30 | 430 | TJ+N (m LOO + 5 CV fits) | 1.7399 |
| tablets | pds | 50 | 410 | single transfer fit + prediction | 0.0227 |
| tablets | pds | 50 | 410 | TJ+N (m LOO + 5 CV fits) | 2.56 |
| mango | sbc | 10 | 715 | single transfer fit + prediction | 0.0004 |
| mango | sbc | 10 | 715 | TJ+N (m LOO + 5 CV fits) | 0.0226 |
| mango | sbc | 20 | 705 | single transfer fit + prediction | 0.0005 |
| mango | sbc | 20 | 705 | TJ+N (m LOO + 5 CV fits) | 0.033 |
| mango | sbc | 30 | 695 | single transfer fit + prediction | 0.0004 |
| mango | sbc | 30 | 695 | TJ+N (m LOO + 5 CV fits) | 0.0425 |
| mango | sbc | 50 | 675 | single transfer fit + prediction | 0.0004 |
| mango | sbc | 50 | 675 | TJ+N (m LOO + 5 CV fits) | 0.0634 |

# Table S8. Normalised Gaussian competitor (E9)

Table S8. TJ+N versus Gaussian leave-one-out intervals without (Gauss-LOO) and with (Gauss-LOO-N) difficulty normalisation (200 repetitions; corn and tablets PDS, mango SBC, mango instruments PDS and SBC).

| Set | Method | α | Mean coverage | Under-covering | Median IS | Median width |
| --- | --- | --- | --- | --- | --- | --- |
| corn | Gauss-LOO | 0.05 | 0.945 | 6/24 | 1.27 | 1.05 |
| corn | Gauss-LOO | 0.1 | 0.903 | 0/24 | 1.07 | 0.863 |
| corn | Gauss-LOO | 0.2 | 0.818 | 0/24 | 0.878 | 0.658 |
| corn | Gauss-LOO-N | 0.05 | 0.951 | 0/24 | 1.26 | 1.09 |
| corn | Gauss-LOO-N | 0.1 | 0.905 | 0/24 | 1.07 | 0.897 |
| corn | Gauss-LOO-N | 0.2 | 0.811 | 0/24 | 0.884 | 0.683 |
| corn | TJ+N | 0.05 | 0.986 | 0/24 | inf | 1.07 |
| corn | TJ+N | 0.1 | 0.932 | 0/24 | 1.15 | 0.996 |
| corn | TJ+N | 0.2 | 0.835 | 0/24 | 0.907 | 0.732 |
| mango | Gauss-LOO | 0.05 | 0.955 | 0/4 | 5.13 | 4.33 |
| mango | Gauss-LOO | 0.1 | 0.902 | 0/4 | 4.45 | 3.59 |
| mango | Gauss-LOO | 0.2 | 0.792 | 1/4 | 3.76 | 2.76 |
| mango | Gauss-LOO-N | 0.05 | 0.956 | 0/4 | 5.05 | 4.29 |
| mango | Gauss-LOO-N | 0.1 | 0.902 | 0/4 | 4.38 | 3.55 |
| mango | Gauss-LOO-N | 0.2 | 0.790 | 2/4 | 3.72 | 2.74 |
| mango | TJ+N | 0.05 | 0.968 | 0/4 | 5.16 | 4.41 |
| mango | TJ+N | 0.1 | 0.902 | 0/4 | 4.4 | 3.6 |
| mango | TJ+N | 0.2 | 0.803 | 0/4 | 3.75 | 2.83 |
| mango_inst | Gauss-LOO | 0.05 | 0.950 | 4/24 | 7.48 | 6.35 |
| mango_inst | Gauss-LOO | 0.1 | 0.908 | 0/24 | 6.46 | 5.23 |
| mango_inst | Gauss-LOO | 0.2 | 0.820 | 0/24 | 5.39 | 4.01 |
| mango_inst | Gauss-LOO-N | 0.05 | 0.958 | 6/24 | 7.88 | 6.67 |
| mango_inst | Gauss-LOO-N | 0.1 | 0.915 | 0/24 | 6.64 | 5.5 |
| mango_inst | Gauss-LOO-N | 0.2 | 0.810 | 3/24 | 5.44 | 4.22 |
| mango_inst | TJ+N | 0.05 | 0.974 | 0/24 | 11.9 | 7.73 |
| mango_inst | TJ+N | 0.1 | 0.919 | 0/24 | 6.97 | 5.74 |
| mango_inst | TJ+N | 0.2 | 0.827 | 0/24 | 5.48 | 4.22 |
| tablets | Gauss-LOO | 0.05 | 0.949 | 6/12 | 20.1 | 16.5 |
| tablets | Gauss-LOO | 0.1 | 0.910 | 0/12 | 17.5 | 13.7 |
| tablets | Gauss-LOO | 0.2 | 0.825 | 1/12 | 14.9 | 10.6 |
| tablets | Gauss-LOO-N | 0.05 | 0.948 | 2/12 | 21.4 | 16 |
| tablets | Gauss-LOO-N | 0.1 | 0.907 | 1/12 | 18.1 | 13.3 |
| tablets | Gauss-LOO-N | 0.2 | 0.819 | 0/12 | 14.8 | 10.3 |
| tablets | TJ+N | 0.05 | 0.971 | 0/12 | 24.7 | 18.4 |
| tablets | TJ+N | 0.1 | 0.912 | 0/12 | 18.2 | 13 |
| tablets | TJ+N | 0.2 | 0.820 | 0/12 | 14.8 | 10 |

# Table S9. Robustness

Table S9. Robustness experiment E4 (α = 0.10; corn m = 15, tablets assay and mango m = 20; 200 repetitions): mean and minimum coverage over scenarios, median interval score and under-covering scenarios.

| dataset | variant | method | coverage | coverage_min | IS_median | n_under | n_scen |
| --- | --- | --- | --- | --- | --- | --- | --- |
| corn | half_source | ASTM-PI | 0.335 | 0.071 | 2.841 | 4 | 4 |
| corn | half_source | Gauss-LOO | 0.908 | 0.898 | 1.092 | 0 | 4 |
| corn | half_source | SCP-S | 0.608 | 0.157 | 1.627 | 4 | 4 |
| corn | half_source | SCP-T-split-N | 1.0 | 1.0 | inf | 0 | 4 |
| corn | half_source | TJ+ | 0.947 | 0.941 | 1.183 | 0 | 4 |
| corn | half_source | TJ+N | 0.948 | 0.932 | 1.2 | 0 | 4 |
| corn | half_source | WCP-S | 0.7 | 0.355 | inf | 4 | 4 |
| corn | ks | ASTM-PI | 0.232 | 0.033 | 2.929 | 4 | 4 |
| corn | ks | Gauss-LOO | 0.913 | 0.9 | 1.048 | 0 | 4 |
| corn | ks | SCP-S | 0.452 | 0.067 | 1.813 | 4 | 4 |
| corn | ks | SCP-T-split-N | 1.0 | 1.0 | inf | 0 | 4 |
| corn | ks | TJ+ | 0.955 | 0.946 | 1.191 | 0 | 4 |
| corn | ks | TJ+N | 0.947 | 0.934 | 1.185 | 0 | 4 |
| corn | ks | WCP-S | 0.554 | 0.221 | inf | 4 | 4 |
| corn | noise_100 | ASTM-PI | 0.181 | 0.033 | 3.558 | 4 | 4 |
| corn | noise_100 | Gauss-LOO | 0.908 | 0.893 | 1.167 | 0 | 4 |
| corn | noise_100 | SCP-S | 0.327 | 0.058 | 3.197 | 4 | 4 |
| corn | noise_100 | SCP-T-split-N | 1.0 | 1.0 | inf | 0 | 4 |
| corn | noise_100 | TJ+ | 0.956 | 0.946 | 1.306 | 0 | 4 |
| corn | noise_100 | TJ+N | 0.956 | 0.939 | 1.261 | 0 | 4 |
| corn | noise_100 | WCP-S | 0.473 | 0.238 | inf | 4 | 4 |
| corn | noise_1000 | ASTM-PI | 0.234 | 0.037 | 2.983 | 4 | 4 |
| corn | noise_1000 | Gauss-LOO | 0.905 | 0.892 | 1.044 | 0 | 4 |
| corn | noise_1000 | SCP-S | 0.456 | 0.066 | 1.794 | 4 | 4 |
| corn | noise_1000 | SCP-T-split-N | 1.0 | 1.0 | inf | 0 | 4 |
| corn | noise_1000 | TJ+ | 0.951 | 0.943 | 1.17 | 0 | 4 |
| corn | noise_1000 | TJ+N | 0.946 | 0.934 | 1.175 | 0 | 4 |
| corn | noise_1000 | WCP-S | 0.554 | 0.221 | inf | 4 | 4 |
| corn | noise_300 | ASTM-PI | 0.23 | 0.036 | 3.006 | 4 | 4 |
| corn | noise_300 | Gauss-LOO | 0.906 | 0.895 | 1.054 | 0 | 4 |
| corn | noise_300 | SCP-S | 0.446 | 0.065 | 1.97 | 4 | 4 |
| corn | noise_300 | SCP-T-split-N | 1.0 | 1.0 | inf | 0 | 4 |
| corn | noise_300 | TJ+ | 0.952 | 0.944 | 1.175 | 0 | 4 |
| corn | noise_300 | TJ+N | 0.945 | 0.931 | 1.168 | 0 | 4 |
| corn | noise_300 | WCP-S | 0.548 | 0.22 | inf | 4 | 4 |
| corn | shift_-1 | ASTM-PI | 0.086 | 0.0 | 18.75 | 4 | 4 |
| corn | shift_-1 | Gauss-LOO | 0.17 | 0.025 | 13.629 | 4 | 4 |
| corn | shift_-1 | SCP-S | 0.074 | 0.0 | 19.4 | 4 | 4 |
| corn | shift_-1 | SCP-T-split-N | 1.0 | 1.0 | inf | 0 | 4 |
| corn | shift_-1 | TJ+ | 0.286 | 0.058 | 9.399 | 4 | 4 |
| corn | shift_-1 | TJ+N | 0.843 | 0.679 | 5.656 | 2 | 4 |
| corn | shift_-1 | WCP-S | 0.416 | 0.361 | 30.404 | 4 | 4 |
| corn | shift_1 | ASTM-PI | 0.079 | 0.0 | 19.718 | 4 | 4 |
| corn | shift_1 | Gauss-LOO | 0.151 | 0.02 | 14.643 | 4 | 4 |
| corn | shift_1 | SCP-S | 0.067 | 0.0 | 20.582 | 4 | 4 |
| corn | shift_1 | SCP-T-split-N | 1.0 | 1.0 | inf | 0 | 4 |
| corn | shift_1 | TJ+ | 0.262 | 0.05 | 10.341 | 4 | 4 |
| corn | shift_1 | TJ+N | 0.823 | 0.605 | 5.795 | 2 | 4 |
| corn | shift_1 | WCP-S | 0.497 | 0.416 | inf | 4 | 4 |
| mango | half_source | ASTM-PI | 0.755 | 0.755 | 4.864 | 1 | 1 |
| mango | half_source | Gauss-LOO | 0.899 | 0.899 | 4.46 | 0 | 1 |
| mango | half_source | SCP-S | 0.752 | 0.752 | 4.887 | 1 | 1 |
| mango | half_source | SCP-T-split-N | 0.905 | 0.905 | 4.857 | 0 | 1 |
| mango | half_source | TJ+ | 0.9 | 0.9 | 4.51 | 0 | 1 |
| mango | half_source | TJ+N | 0.899 | 0.899 | 4.479 | 0 | 1 |
| mango | half_source | WCP-S | 0.785 | 0.785 | inf | 1 | 1 |
| mango | ks | ASTM-PI | 0.752 | 0.752 | 4.888 | 1 | 1 |
| mango | ks | Gauss-LOO | 0.968 | 0.968 | 4.994 | 0 | 1 |
| mango | ks | SCP-S | 0.733 | 0.733 | 5.028 | 1 | 1 |
| mango | ks | SCP-T-split-N | 0.956 | 0.956 | 5.337 | 0 | 1 |
| mango | ks | TJ+ | 0.968 | 0.968 | 5.005 | 0 | 1 |
| mango | ks | TJ+N | 0.964 | 0.964 | 4.958 | 0 | 1 |
| mango | ks | WCP-S | 0.739 | 0.739 | inf | 1 | 1 |
| mango | noise_100 | ASTM-PI | 0.508 | 0.508 | 13.075 | 1 | 1 |
| mango | noise_100 | Gauss-LOO | 0.895 | 0.895 | 7.863 | 0 | 1 |
| mango | noise_100 | SCP-S | 0.49 | 0.49 | 13.545 | 1 | 1 |
| mango | noise_100 | SCP-T-split-N | 0.91 | 0.91 | 8.261 | 0 | 1 |
| mango | noise_100 | TJ+ | 0.9 | 0.9 | 7.968 | 0 | 1 |
| mango | noise_100 | TJ+N | 0.901 | 0.901 | 7.627 | 0 | 1 |
| mango | noise_100 | WCP-S | 0.507 | 0.507 | inf | 1 | 1 |
| mango | noise_1000 | ASTM-PI | 0.744 | 0.744 | 5.038 | 1 | 1 |
| mango | noise_1000 | Gauss-LOO | 0.9 | 0.9 | 4.554 | 0 | 1 |
| mango | noise_1000 | SCP-S | 0.726 | 0.726 | 5.186 | 1 | 1 |
| mango | noise_1000 | SCP-T-split-N | 0.908 | 0.908 | 4.957 | 0 | 1 |
| mango | noise_1000 | TJ+ | 0.902 | 0.902 | 4.63 | 0 | 1 |
| mango | noise_1000 | TJ+N | 0.902 | 0.902 | 4.54 | 0 | 1 |
| mango | noise_1000 | WCP-S | 0.733 | 0.733 | inf | 1 | 1 |
| mango | noise_300 | ASTM-PI | 0.693 | 0.693 | 6.271 | 1 | 1 |
| mango | noise_300 | Gauss-LOO | 0.9 | 0.9 | 5.235 | 0 | 1 |
| mango | noise_300 | SCP-S | 0.675 | 0.675 | 6.478 | 1 | 1 |
| mango | noise_300 | SCP-T-split-N | 0.91 | 0.91 | 5.722 | 0 | 1 |
| mango | noise_300 | TJ+ | 0.907 | 0.907 | 5.328 | 0 | 1 |
| mango | noise_300 | TJ+N | 0.908 | 0.908 | 5.149 | 0 | 1 |
| mango | noise_300 | WCP-S | 0.685 | 0.685 | inf | 1 | 1 |
| mango | shift_-1 | ASTM-PI | 0.023 | 0.023 | 50.584 | 1 | 1 |
| mango | shift_-1 | Gauss-LOO | 0.07 | 0.07 | 42.187 | 1 | 1 |
| mango | shift_-1 | SCP-S | 0.018 | 0.018 | 52.878 | 1 | 1 |
| mango | shift_-1 | SCP-T-split-N | 0.789 | 0.789 | 13.758 | 1 | 1 |
| mango | shift_-1 | TJ+ | 0.081 | 0.081 | 41.036 | 1 | 1 |
| mango | shift_-1 | TJ+N | 0.732 | 0.732 | 13.493 | 1 | 1 |
| mango | shift_-1 | WCP-S | 0.305 | 0.305 | inf | 1 | 1 |
| mango | shift_1 | ASTM-PI | 0.109 | 0.109 | 30.41 | 1 | 1 |
| mango | shift_1 | Gauss-LOO | 0.206 | 0.206 | 25.282 | 1 | 1 |
| mango | shift_1 | SCP-S | 0.069 | 0.069 | 34.439 | 1 | 1 |
| mango | shift_1 | SCP-T-split-N | 0.97 | 0.97 | 15.736 | 0 | 1 |
| mango | shift_1 | TJ+ | 0.216 | 0.216 | 24.705 | 1 | 1 |
| mango | shift_1 | TJ+N | 0.974 | 0.974 | 14.14 | 0 | 1 |
| mango | shift_1 | WCP-S | 0.207 | 0.207 | inf | 1 | 1 |
| tablets | half_source | ASTM-PI | 0.971 | 0.971 | 21.998 | 0 | 1 |
| tablets | half_source | Gauss-LOO | 0.916 | 0.916 | 20.21 | 0 | 1 |
| tablets | half_source | SCP-S | 0.957 | 0.957 | 20.65 | 0 | 1 |
| tablets | half_source | SCP-T-split-N | 0.913 | 0.913 | 22.635 | 0 | 1 |
| tablets | half_source | TJ+ | 0.908 | 0.908 | 20.01 | 0 | 1 |
| tablets | half_source | TJ+N | 0.907 | 0.907 | 19.729 | 0 | 1 |
| tablets | half_source | WCP-S | 0.964 | 0.964 | 21.402 | 0 | 1 |
| tablets | ks | ASTM-PI | 0.94 | 0.94 | 21.237 | 0 | 1 |
| tablets | ks | Gauss-LOO | 0.954 | 0.954 | 23.123 | 0 | 1 |
| tablets | ks | SCP-S | 0.922 | 0.922 | 21.252 | 0 | 1 |
| tablets | ks | SCP-T-split-N | 0.92 | 0.92 | 25.461 | 0 | 1 |
| tablets | ks | TJ+ | 0.956 | 0.956 | 23.808 | 0 | 1 |
| tablets | ks | TJ+N | 0.93 | 0.93 | 20.831 | 0 | 1 |
| tablets | ks | WCP-S | 0.928 | 0.928 | 21.95 | 0 | 1 |
| tablets | noise_100 | ASTM-PI | 0.927 | 0.927 | 21.563 | 0 | 1 |
| tablets | noise_100 | Gauss-LOO | 0.91 | 0.91 | 23.167 | 0 | 1 |
| tablets | noise_100 | SCP-S | 0.91 | 0.91 | 21.729 | 0 | 1 |
| tablets | noise_100 | SCP-T-split-N | 0.913 | 0.913 | 28.094 | 0 | 1 |
| tablets | noise_100 | TJ+ | 0.913 | 0.913 | 23.109 | 0 | 1 |
| tablets | noise_100 | TJ+N | 0.914 | 0.914 | 22.03 | 0 | 1 |
| tablets | noise_100 | WCP-S | 0.926 | 0.926 | 23.533 | 0 | 1 |
| tablets | noise_1000 | ASTM-PI | 0.928 | 0.928 | 21.591 | 0 | 1 |
| tablets | noise_1000 | Gauss-LOO | 0.91 | 0.91 | 23.064 | 0 | 1 |
| tablets | noise_1000 | SCP-S | 0.91 | 0.91 | 21.695 | 0 | 1 |
| tablets | noise_1000 | SCP-T-split-N | 0.916 | 0.916 | 27.687 | 0 | 1 |
| tablets | noise_1000 | TJ+ | 0.913 | 0.913 | 23.01 | 0 | 1 |
| tablets | noise_1000 | TJ+N | 0.916 | 0.916 | 22.036 | 0 | 1 |
| tablets | noise_1000 | WCP-S | 0.927 | 0.927 | 23.478 | 0 | 1 |
| tablets | noise_300 | ASTM-PI | 0.927 | 0.927 | 21.568 | 0 | 1 |
| tablets | noise_300 | Gauss-LOO | 0.91 | 0.91 | 23.077 | 0 | 1 |
| tablets | noise_300 | SCP-S | 0.91 | 0.91 | 21.718 | 0 | 1 |
| tablets | noise_300 | SCP-T-split-N | 0.916 | 0.916 | 27.932 | 0 | 1 |
| tablets | noise_300 | TJ+ | 0.912 | 0.912 | 23.032 | 0 | 1 |
| tablets | noise_300 | TJ+N | 0.915 | 0.915 | 22.011 | 0 | 1 |
| tablets | noise_300 | WCP-S | 0.926 | 0.926 | 23.397 | 0 | 1 |
| tablets | shift_-1 | ASTM-PI | 0.02 | 0.02 | 304.33 | 1 | 1 |
| tablets | shift_-1 | Gauss-LOO | 0.035 | 0.035 | 300.236 | 1 | 1 |
| tablets | shift_-1 | SCP-S | 0.015 | 0.015 | 314.736 | 1 | 1 |
| tablets | shift_-1 | SCP-T-split-N | 0.253 | 0.253 | 226.108 | 1 | 1 |
| tablets | shift_-1 | TJ+ | 0.034 | 0.034 | 286.928 | 1 | 1 |
| tablets | shift_-1 | TJ+N | 0.069 | 0.069 | 259.707 | 1 | 1 |
| tablets | shift_-1 | WCP-S | 0.022 | 0.022 | 340.682 | 1 | 1 |
| tablets | shift_1 | ASTM-PI | 0.008 | 0.008 | 350.126 | 1 | 1 |
| tablets | shift_1 | Gauss-LOO | 0.018 | 0.018 | 347.84 | 1 | 1 |
| tablets | shift_1 | SCP-S | 0.007 | 0.007 | 360.886 | 1 | 1 |
| tablets | shift_1 | SCP-T-split-N | 0.22 | 0.22 | 245.495 | 1 | 1 |
| tablets | shift_1 | TJ+ | 0.014 | 0.014 | 341.348 | 1 | 1 |
| tablets | shift_1 | TJ+N | 0.04 | 0.04 | 299.11 | 1 | 1 |
| tablets | shift_1 | WCP-S | 0.015 | 0.015 | 444.978 | 1 | 1 |

# Table S10. Sub-channel drift

Table S10. Coverage under a ±0.25-channel wavelength shift of the test spectra after standardisation (α = 0.10; 200 repetitions).

| dataset | shift | method | coverage | coverage_min | under | n |
| --- | --- | --- | --- | --- | --- | --- |
| corn | shift_-0.25 | ASTM-PI | 0.152 | 0.017 | 4 | 4 |
| corn | shift_-0.25 | Gauss-LOO | 0.692 | 0.672 | 4 | 4 |
| corn | shift_-0.25 | SCP-S | 0.259 | 0.025 | 4 | 4 |
| corn | shift_-0.25 | SCP-T-split-N | 1.0 | 1.0 | 0 | 4 |
| corn | shift_-0.25 | TJ+ | 0.807 | 0.781 | 4 | 4 |
| corn | shift_-0.25 | TJ+N | 0.928 | 0.879 | 0 | 4 |
| corn | shift_-0.25 | WCP-S | 0.459 | 0.281 | 4 | 4 |
| corn | shift_0.25 | ASTM-PI | 0.152 | 0.021 | 4 | 4 |
| corn | shift_0.25 | Gauss-LOO | 0.665 | 0.616 | 4 | 4 |
| corn | shift_0.25 | SCP-S | 0.251 | 0.03 | 4 | 4 |
| corn | shift_0.25 | SCP-T-split-N | 1.0 | 1.0 | 0 | 4 |
| corn | shift_0.25 | TJ+ | 0.781 | 0.746 | 4 | 4 |
| corn | shift_0.25 | TJ+N | 0.919 | 0.839 | 1 | 4 |
| corn | shift_0.25 | WCP-S | 0.501 | 0.392 | 4 | 4 |
| mango | shift_-0.25 | ASTM-PI | 0.624 | 0.624 | 1 | 1 |
| mango | shift_-0.25 | Gauss-LOO | 0.781 | 0.781 | 1 | 1 |
| mango | shift_-0.25 | SCP-S | 0.605 | 0.605 | 1 | 1 |
| mango | shift_-0.25 | SCP-T-split-N | 0.885 | 0.885 | 0 | 1 |
| mango | shift_-0.25 | TJ+ | 0.786 | 0.786 | 1 | 1 |
| mango | shift_-0.25 | TJ+N | 0.861 | 0.861 | 1 | 1 |
| mango | shift_-0.25 | WCP-S | 0.619 | 0.619 | 1 | 1 |
| mango | shift_0.25 | ASTM-PI | 0.682 | 0.682 | 1 | 1 |
| mango | shift_0.25 | Gauss-LOO | 0.833 | 0.833 | 1 | 1 |
| mango | shift_0.25 | SCP-S | 0.659 | 0.659 | 1 | 1 |
| mango | shift_0.25 | SCP-T-split-N | 0.958 | 0.958 | 0 | 1 |
| mango | shift_0.25 | TJ+ | 0.832 | 0.832 | 1 | 1 |
| mango | shift_0.25 | TJ+N | 0.961 | 0.961 | 0 | 1 |
| mango | shift_0.25 | WCP-S | 0.682 | 0.682 | 1 | 1 |
| tablets | shift_-0.25 | ASTM-PI | 0.663 | 0.663 | 1 | 1 |
| tablets | shift_-0.25 | Gauss-LOO | 0.659 | 0.659 | 1 | 1 |
| tablets | shift_-0.25 | SCP-S | 0.613 | 0.613 | 1 | 1 |
| tablets | shift_-0.25 | SCP-T-split-N | 0.697 | 0.697 | 1 | 1 |
| tablets | shift_-0.25 | TJ+ | 0.668 | 0.668 | 1 | 1 |
| tablets | shift_-0.25 | TJ+N | 0.591 | 0.591 | 1 | 1 |
| tablets | shift_-0.25 | WCP-S | 0.67 | 0.67 | 1 | 1 |
| tablets | shift_0.25 | ASTM-PI | 0.661 | 0.661 | 1 | 1 |
| tablets | shift_0.25 | Gauss-LOO | 0.658 | 0.658 | 1 | 1 |
| tablets | shift_0.25 | SCP-S | 0.613 | 0.613 | 1 | 1 |
| tablets | shift_0.25 | SCP-T-split-N | 0.726 | 0.726 | 1 | 1 |
| tablets | shift_0.25 | TJ+ | 0.675 | 0.675 | 1 | 1 |
| tablets | shift_0.25 | TJ+N | 0.615 | 0.615 | 1 | 1 |
| tablets | shift_0.25 | WCP-S | 0.671 | 0.671 | 1 | 1 |

# Fig. S1. CNN training

Fig. S1. Training and validation loss of the 1D-CNN (mango, seasons 1–3).

# Fig. S2. Friedman ranks

Fig. S2. Mean Friedman ranks of the scenario median interval score over the 40 primary scenarios (1 = best).

# Fig. S3. Computational cost

Fig. S3. Wall-clock time per test batch (single CPU core, median of 20 runs, logarithmic scale).